QUICK REFERENCE

OVERVIEW

PROGRAM MODE ACCESS

ENTERING A PROGRAM COMMAND

EXITING THE PROGRAM MODE

USER PROGRAMMING COMMANDS

User Numbers versus CTCSS Tones

User Numbers versus DCS Codes

SYSTEM PROGRAMMING COMMANDS

DIAGNOSTIC PROGRAMMING COMMANDS

## OVERVIEW

This quick reference is provided as convenience for experienced technicians. Very little is explained here. If information provided here is not clear, refer to Section 4 for more complete descriptions.

## PROGRAM MODE ACCESS

The programming mode can be accessed by entering the Program Mode Access Code over the radio. The default value for the Program Mode Access Code is “12037”. The Model 37-MAX will respond to decoding a valid access code by transmitting a “go-ahead” tone consisting of five short beeps. If the access code decoded by the 37-MAX is invalid, then the panel will ignore the attempt.

## ENTERING A PROGRAM COMMAND

To enter a programming command, a DTMF number is entered followed by the “#” key. Once the “#” has been entered, the Model 37-MAX will respond either with the 5-beep “go-ahead” prompt to indicate the command was accepted, or the high-low “error tone” sequence to indicate that an invalid command sequence was received.

Some commands will require additional numbers be entered. When it receives a command of this type, the 37-MAX will respond with two quick beeps to indicate that it is waiting for the additional data. In most cases, the data will be followed by a “#”.

The Model 37-MAX accepts its commands and updates its database in “real time”. The programming mode can be exited at any time and the panel will “remember” any and all changes made up to that point.

The commands should all be entered one at a time, do not try to string several commands together in a single transmission. If the next command is not received within 60 seconds of the last command, the 37-MAX will exit the programming mode and return itself to the normal, idle condition.

## EXITING THE PROGRAM MODE

When a programming session is completed, the programming mode is exited by entering “99#”. The 37-MAX will transmit a prompt that sounds like the ringing of an electronic telephone, and then return to its normal, idle state.

If the Model 37-MAX receives no new commands for a period of 60 seconds, it will exit the program mode and return to the normal idle state on its own.

## USER PROGRAMMING COMMANDS

110#  uu#	Disable User Number “uu” (default) 
111#  uu#	Enable User Number “uu”

122#  uu#  ee#	Tone Translation (uu = user, ee = encode to use)
			(default, encode is the same as decode)

120#  uu#	Encode Disabled During TX-Hold for User “uu” (default) 
121#  uu#	Encode Enabled During TX-Hold for User “uu”

123#  uu#	Use Normal TX-Hold Timer for User “uu” (default)
124#  uu#	Use Alternate TX-Hold Timer for User “uu”

130#  uu#	Reserve Mode Disabled for User “uu” (default) 
131#  uu#	Reserve Mode Enabled for User “uu”

150#  uu#	Courtesy Tone Disabled for User “uu” (default) 
151#  uu#	Courtesy Tone Enabled for User “uu”

99#	Exit Program Mode

### User Numbers versus CTCSS Tones

| User # | Tone | Motorola Reed Code | User # | Tone | Motorola Reed Code | User # | Tone | Motorola Reed Code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Carrier Repeat |  | 17 | 114.8 | 2A | 34 | 179.9 | 6B |
| 1 | 67.0 | XZ | 18 | 118.8 | 2B | 35 | 183.5 | J5 |
| 2 | 69.4 | WZ | 19 | 123.0 | 3Z | 36 | 186.2 | 7Z |
| 3 | 71.9 | XA | 20 | 127.3 | 3A | 37 | 189.9 | J6 |
| 4 | 74.4 | WA | 21 | 131.8 | 3B | 38 | 192.8 | 7A |
| 5 | 77.0 | XB | 22 | 136.5 | 4Z | 39 | 196.6 | J7 |
| 6 | 79.7 | WB | 23 | 141.3 | 4A | 40 | 199.5 | J8 |
| 7 | 82.5 | YZ | 24 | 146.2 | 4B | 41 | 203.5 | M1 |
| 8 | 85.4 | YA | 25 | 151.4 | 5Z | 42 | 206.5 | 8Z |
| 9 | 88.5 | YB | 26 | 156.7 | 5A | 43 | 210.7 | M2 |
| 10 | 91.5 | ZZ | 27 | 159.8 | J1 | 44 | 218.1 | M3 |
| 11 | 94.8 | ZA | 28 | 162.2 | 5B | 45 | 225.7 | M4 |
| 12 | 97.4 | ZB | 29 | 165.5 | J2 | 46 | 229.1 | 9Z |
| 13 | 100.0 | 1Z | 30 | 167.9 | 6Z | 47 | 233.6 | M5 |
| 14 | 103.5 | 1A | 31 | 171.3 | J3 | 48 | 241.8 | M6 |
| 15 | 107.2 | 1B | 32 | 173.8 | 6A | 49 | 250.3 | M7 |
| 16 | 110.9 | 2Z | 33 | 177.3 | J4 | 50 | 254.1 | J9 |

### User Numbers versus DCS Codes

| User # | DCS Code | Invert. | User # | DCS Code | Invert. | User # | DCS Code | Invert. | User # | DCS Code | Invert. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 023 | 047 | 77 | 152 | 115 | 103 | 311 | 664 | 129 | 466 | 662 |
| 52 | 025 | 244 | 78 | 155 | 731 | 104 | 315 | 423 | 130 | 503 | 162 |
| 53 | 026 | 464 | 79 | 156 | 265 | 105 | 325 | 526 | 131 | 506 | 073 |
| 54 | 031 | 627 | 80 | 162 | 503 | 106 | 331 | 465 | 132 | 516 | 432 |
| 55 | 032 | 051 | 81 | 165 | 251 | 107 | 332 | 455 | 133 | 523 | 246 |
| 56 | 036 | 172 | 82 | 172 | 036 | 108 | 343 | 532 | 134 | 526 | 325 |
| 57 | 043 | 445 | 83 | 174 | 074 | 109 | 346 | 612 | 135 | 532 | 343 |
| 58 | 047 | 023 | 84 | 205 | 263 | 110 | 351 | 243 | 136 | 546 | 132 |
| 59 | 051 | 032 | 85 | 212 | 356 | 111 | 356 | 212 | 137 | 565 | 703 |
| 60 | 053 | 452 | 86 | 223 | 134 | 112 | 364 | 131 | 138 | 606 | 631 |
| 61 | 054 | 413 | 87 | 225 | 122 | 113 | 365 | 125 | 139 | 612 | 346 |
| 62 | 065 | 271 | 88 | 226 | 411 | 114 | 371 | 734 | 140 | 624 | 632 |
| 63 | 071 | 306 | 89 | 243 | 351 | 115 | 411 | 226 | 141 | 627 | 031 |
| 64 | 072 | 245 | 90 | 244 | 025 | 116 | 412 | 143 | 142 | 631 | 606 |
| 65 | 073 | 506 | 91 | 245 | 072 | 117 | 413 | 054 | 143 | 632 | 624 |
| 66 | 074 | 174 | 92 | 246 | 523 | 118 | 423 | 315 | 144 | 654 | 743 |
| 67 | 114 | 712 | 93 | 251 | 165 | 119 | 431 | 723 | 145 | 662 | 466 |
| 68 | 115 | 152 | 94 | 252 | 426 | 120 | 432 | 516 | 146 | 664 | 311 |
| 69 | 116 | 754 | 95 | 255 | 446 | 121 | 445 | 043 | 147 | 703 | 565 |
| 70 | 122 | 225 | 96 | 261 | 732 | 122 | 446 | 255 | 148 | 712 | 114 |
| 71 | 125 | 365 | 97 | 263 | 205 | 123 | 452 | 053 | 149 | 723 | 431 |
| 72 | 131 | 364 | 98 | 265 | 156 | 124 | 454 | 266 | 150 | 731 | 155 |
| 73 | 132 | 546 | 99 | 266 | 454 | 125 | 455 | 332 | 151 | 732 | 261 |
| 74 | 134 | 223 | 100 | 271 | 065 | 126 | 462 | 252 | 152 | 734 | 371 |
| 75 | 143 | 412 | 101 | 274 | 145 | 127 | 464 | 026 | 153 | 743 | 654 |
| 76 | 145 | 274 | 102 | 306 | 071 | 128 | 465 | 331 | 154 | 754 | 116 |

## SYSTEM PROGRAMMING COMMANDS

210#  n#	Sets Normal TX-Hold Timer (0 to 9 seconds, default = 2)
211#  n#	Sets Alternate TX-Hold Timer (0 to 9 seconds, default = 3)

220#  n#	Repeater Time-out Timer (1 to 9 minutes, default = 3)

230#		Station ID Disabled 
232#		Station ID Only After Channel Activity (default) 
233#		Station ID At Regular Intervals  
235#  nn#	Set Station ID Interval (1 to 90 minutes, default = 15)  
273#		Station ID Sent at 22 words per minute  
274#		Station ID Sent at 11 words per minute

272#  nn  nn ... #	Enter Station ID Call Sign (default is blank, To clear a call sign 			from memory, enter the command with no digits after it. 272#  #)

| Digits | # | Code | Digits | Letter | Code | Digits | Letter | Code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 0 | - - - - - | 12 | A | · - | 26 | N | - · |
| 01 | 1 | · - - - - | 22 | B | - · · · | 36 | O | - - - |
| 02 | 2 | · · - - - | 32 | C | - · - · | 17 | P | · - - · |
| 03 | 3 | · · · - - | 13 | D | - · · | 10 | Q | - - · - |
| 04 | 4 | · · · · - | 23 | E | · | 27 | R | · - · |
| 05 | 5 | · · · · · | 33 | F | · · - · | 37 | S | · · · |
| 06 | 6 | - · · · · | 14 | G | - - · | 18 | T | - |
| 07 | 7 | - - · · · | 24 | H | · · · · | 28 | U | · · - |
| 08 | 8 | - - - · · | 34 | I | · · | 38 | V | · · · - |
| 09 | 9 | - - - - · | 15 | J | · - - - | 19 | W | · - - |
|  |  |  | 25 | K | - · - | 29 | X | - · · - |
| 30 | / | - · · - · | 35 | L | · - · · | 39 | Y | - · - - |
| # | END |  | 16 | M | - - | 20 | Z | - - · · |

244#		Set Courtesy Tone Frequency to 500 Hz
245#		Set Courtesy Tone Frequency to 1000 Hz (default)
246#		Set Courtesy Tone Frequency to 1500Hz

290#  nnnnn#	Set Program Mode Access Code to “nnnnn”

215#		DCS Encode set to Normal Polarity (default)
216#		DCS Encode set to Inverted Polarity

217#		DCS Decode set to Normal Polarity (default)
218#		DCS Decode set to Inverted Polarity

## DIAGNOSTIC PROGRAMMING COMMANDS

300#		Repeater Knockdown
301#		Repeater Setup

391#		Reset all Programming to Factory Default Values

392#		Transmit Audio Test

393#		Transmit Audio Pre-Emphasis Test

394#		Repeat Audio Test

395#		CTCSS Encode Test

396#		CTCSS Frequency Response Test

397#		DCS Encode Test

99#		Exit Program Mode