PM200/250

Programming Manual

# Contents

Introduction	6

Programmer Installation	7

Programming	7

Help files and Language settings	8

Tool Bar	9

Icon	9

Programming Mode	10

Reading	10

Writing	10

Cloning	10

Standard non-selcall parameters	11

Radio Parameters	11

LCD Logo	11

Speaker	11

Power on Channel	12

Select radio parameters	13

Frequency	13

CTCSS	14

DCS	15

Contents

Channel Spacing	15

Group Setting	15

Clear One Channel	16

Clear All Channels	16

TX Power	16

Options	17

Programmable Function Keys	17

Power	17

Light	18

Monitor	18

Lock	18

Scan	18

Public Address	18

Open/Close	18

Selcall	18

SMS	18

1200/2400	18

DTMF	18

Group	18

Asic	18

Settings and Timings	19

PTT Timer	20

TX Timer	20

TX Inhibit	20

TX Inhibit Alert	20

Open Lock	21

Busy Lockout	21

Mark Idle	21

TX Delay	21

Monitor	21

Hook	21

Hook Scan	22

Beep	22

Public Address	22

Power Up Alert	22

Power up	22

Contents

Auto Mute	22

Auto Mute Timer	22

Scanning Parameters	23

Scanning examples	23

Priority Look Back Scan	24

Scan channel delete	25

Group Scan edit mode	25

Priority scan channel edit	25

## Selcall tones and sequences	27

Tone sets	27

Sequence	28

Tone Length	28

Lead In	28

1st Tone	28

Lead Out	28

Encode	29

Emergency Sequence	29

Check Open	29

Check Busy	29

Encode Alert	30

Repeats	30

Cycle Repeat Time	30

Transmit Timer	30

ANI	30

ANI Sequence	30

ANI at	30

Multi ANI Delay	30

Call1 /Call2/Call3 Sequence	30

Check Open	31

Check Busy	31

Encode Alert	31

Ack.1 to Ack.4	31

Decoder	32

Call1 /Call2/Call3	32

Control 1 to 4	32

Contents

Action	33

Close	33

Open	33

Stun	33

Revive	33

Covert On	33

Covert Off	33

Acknowledge	34

Selcall Text	34

Group Call	34

Decoder Tone Length	34

Covert Timer	34

TX Timer	34

RX Timer	34

## DTMF and Selcall Status	35

Selcall Status	35

DTMF	36

DTMF lead in time	36

DTMF lead Out Time	36

DTMF Mark Time	36

DTMF Address	36

## MODEM - SMS	37

SMS	37

TX On Delay	37

TX Off Delay	37

Contact Details	38

Note:

Introduction

Although this manual is a generic manual for PM200 and PM250 there are a number of differences between PM200 and PM250. A separate appendix has been written which includes the drawings and part list for the PM250. Please be aware that all drawings and parts are only relevant to the PM200. All features and functions are relevant to both models.  The PM250 appendix can be downloaded from the MaxonCIC website http://www.maxoncic.co.uk/

Contents

# Introduction

Introduction

The PM200 Programming Manual is to be used in conjunction with the PM200 programming software and programming lead (PK –PM200-250-01). Combined these items will program the PM200 personality. The Selcall, DTMF, SMS and data features, support a wide range of signaling and control functions including:

- Fully programmable tone sets- including EEA, CCIR and ZVEI

- Automatic Number Identification (ANI) on PTT

- Emergency Calling

- Stun and Revive

- Group Calling

- Covert listening

- SMS (Short message Service)

- Data Facilities Socket

- Programmer definable Keys

- Optional Penalty Modes and PTT Time Out

- DTMF Status Transmission

The PM200 is programmable via the 15 way D-type socket at the rear of the radio using the PM200 Programming Kit.  This is made up of PM200 programming software, and the programming lead PK-PM200-250-01.

The software is best downloaded from the Maxon CIC EUROPE Web-site (http://www.maxoncic.co.uk). The software is an XP Windows based package.

### Operating System

Microsoft Windows 98, 2000, NT, XP

Programmer Installation & Programming

Programmer Installation

Your PM200 programmer is best downloaded from the Maxon web-site, but if you have difficulty downloading the software then you can obtain a copy of the software through Sales.  The software comes on a CD-ROM disk.  Insert the disk into your CD drive.  Click on ‘Start’; click on ‘Run’.  Type,‘d:\setup’ or click on ‘Browse’ and then look for the setup file on your CD drive.  Double click on the Set-up icon to install.  Follow on-screen instructions.

# Programming

This programmer will program the radio parameters and the Selcall features without the need for cable change or software change. All necessary features can be accessed from the front window via Tabs.

To open the programmer, double-click on the PM200 icon in the Start Menu Taskbar.

The PM200 Programming utilizes COM 1 on the PC as the default Programming Port. This can be changed under the “Program (P)” “Port (P)”, along the Tool bar. See below: -

Setup of Com Ports

(Fig 1)

Files can be saved, and viewed at a later date, or previously saved files opened, using the “File (F)” Menu along the Tool bar.

Please note that it may prove helpful to create named folders in the PM200 directory, to save files specific to that radio and it’s’ band, i.e. PM200_WITH_SELCALL, etc.

Help files

If the help files fail to initiate please follow the instructions below. (This instruction is for XP only)

## (Fig. 2)

Click on Control panel and double click the Regional and Language

Click on Languages and make sure install East Asian languages are ticked.

Help Files & Language Settings

Then Click Details.

## (Fig. 3)

Click Advanced.

## (Fig. 4)

Make sure the Compatibility Configuration is ticked.

Click on apply and Click OK you may have to restart your PC.

This has now completed the set-up for reading the help files. Please check you can now read the help files.

## (Fig.5)

## Tool Bar

There are 3 items in the top right hand side tool bar, File(F), Program(P) and Help(H). There are seven printing options, Exit, New programming page, Open pre-saved files, Save items and Open ATE file (please note this for engineering use only).

Help Files & Language Settings

(Fig. 6)

## Icon Bar

There are seven icons along the top right hand side (see below).

(Fig.7)

Write

Read

Print

Open File

Help

New File

Save

To Program or to Read from the radio click the “Program (P)” menu along the Tool bar. Click on “Read/write(R)”. Alternatively use the read/write icons programming menu: -

## Programming Mode

To place the PM200 into program mode, connect PK-PM200-250-01 programming lead to the PC and the PM200 (via a 15way High density D-type connector please see rear panel layout Fig1). Switch the PM200 on with the programming lead connected. If the PC-program enable port is active status, the PCU enters programming mode. The PM200 should now display “ProGrAm”

## Reading

PM200 assumes the PC is ready to receive data. When serial Commands are sent through the lead, PM-200 will transmit the data. During this sequence, the Red LED will flash. In PC program Reading Mode, the LCD displays “Prog-r”.

## Writing

Programming

PM200 assumes the PC is ready to transmit data. When serial Commands are sent through the lead the PC then starts the transmission of data. During this sequence, the Green LED flashes. In the PC program Writing Mode, the LCD displays “Prog-w”.

## Cloning

Cloning is a method of programming another radio with the same profile without using a personal computer or laptop. The master radio is the radio with the desired frequencies the slave radio is the radio to be programmed.

- Press and hold ▲ button on power up on the radio which you would like to copy. In this mode, radio will display “MASTER”. The radio is now ready to transmit programming file.

- Press and hold the▼ button on power up on the radio you would like to program. In this mode, radio will display “SLAVE” with green led blinking.

- Connect the cloning cable.

- Press the master radio ▼ button.

- After cloning, the slave radio will display “ TURN OFF” message, and the master radio displays ” MASTER” message on LCD. Turn off both radios.

# Standard Non-Selcall Parameters

## Radio parameters

Standard Non-selcall Parameters

## (Fig.8)

### Radio Band

To select Band click on “Radio band”

UHF2 (440~480MHz)

VHF2 (148~174MHz)

### LCD Display Logo

To change power up Logo, type new message in the LDC display logo box. Maximum of 9 characters.

### Speaker Internal/External

This feature switches the internal/external speaker on and off. Disabled the internal speaker is on, Enabled internal speaker is off. Please note the internal-external speaker can be switched by the user. Press and hold the P1-P3 button on switch on. This will toggle the internal/external speaker.

### Power on channel

Double click power on channel (please note this can only be done if the frequencies are already entered). When activated the radio will power up on selected channel, if not selected, the radio will power up on the last channel used. This parameter is marked “PowON CH”.

Standard Non-selcall Parameters

# Select Radio Parameters

## Frequency

To select the correct frequency double click on the desired channel as per figure 8.

Frequency menu

Select Radio Parameters

## (Fig.9)

Channel and Group numbers are displayed at the top of dialogue box.

Enter TX/RX frequencies using decimal point. If the frequencies are back to back the “RX=TX” will copy the frequency to both settings.

Note: Frequency must be divisible by 6.25 kHz.

## CTCSS

To set the CTCSS (Continuous Tone Coded Squelch System) click on correct RX/TX CTCSS, scroll to correct tone frequency and select by single click. See table for available tones.

CTCSS & CTCSS Table

## (Fig.10)

| No. | Freq. Hz | No. | Freq. Hz | No. | Freq. Hz | No. | Freq. Hz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 67.0 | 11 | 97.4 | 21 | 136.5 | 31 | 192.8 |
| 02 | 71.9 | 12 | 100.0 | 22 | 141.3 | 32 | 203.5 |
| 03 | 74.4 | 13 | 103.5 | 23 | 146.2 | 33 | 210.7 |
| 04 | 77.0 | 14 | 107.2 | 24 | 151.4 | 34 | 218.1 |
| 05 | 79.7 | 15 | 110.9 | 25 | 156.7 | 35 | 225.7 |
| 06 | 82.5 | 16 | 114.8 | 26 | 162.2 | 36 | 233.6 |
| 07 | 85.4 | 17 | 118.8 | 27 | 167.9 | 37 | 241.8 |
| 08 | 88.5 | 18 | 123.0 | 28 | 173.8 | 38 | 250.3 |
| 09 | 91.5 | 19 | 127.3 | 29 | 179.9 |  |  |
| 10 | 94.8 | 20 | 131.8 | 30 | 186.2 |  |  |

CTCSS

(Fig.11)

## DCS & DCS Table

(Fig.12)

To set the DCS (Digital Coded Squelch) click on correct RX/TX DCS, scroll to correct code tone and select by single click. See table for available codes.  For inverse code tick Inverse DCS.

| Octal Code | Octal Code | Octal Code | Octal Code | Octal Code | Octal Code | Octal Code | Octal Code | Octal Code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 023 | 071 | 134 | 223 | 306 | 411 | 503 | 631 | 734 |
| 025 | 072 | 143 | 226 | 311 | 412 | 506 | 632 | 743 |
| 026 | 073 | 152 | 243 | 315 | 413 | 516 | 654 | 754 |
| 031 | 074 | 155 | 244 | 331 | 423 | 532 | 662 |  |
| 032 | 114 | 156 | 245 | 343 | 431 | 546 | 664 |  |
| 043 | 115 | 162 | 251 | 346 | 432 | 565 | 703 |  |
| 047 | 116 | 165 | 261 | 351 | 445 | 606 | 712 |  |
| 051 | 125 | 172 | 263 | 364 | 464 | 612 | 723 |  |
| 054 | 131 | 174 | 265 | 365 | 465 | 624 | 731 |  |
| 065 | 132 | 205 | 271 | 371 | 466 | 627 | 732 |  |

## (Fig. 13)

DCS

Channel Spacing

Select the correct channel spacing by double clicking the Bandwidth parameter.
S = 25 kHz channel spacing. 
N = 12.5 kHz channel spacing.

## Group Setting

To select a group single click on Group setting.

Group (Fig.14)

Group channel numbers (Fig.15)

Double click the correct group number.

Enter the number of channels required in the capability box and press OK.

Group number (Fig.16)

Once frequencies have been added the number of channel will show in the Use boxes.

## Clear One Channel

To clear one channel, highlight channel single click clear channel

## Clear All Channels

To clear all channels

Group Setting

TX Power

There are two settings for TX power high and low
Low = 2 Watts
High = 25 Watts

# Options

## Programmable Function Keys

(Fig.17)

Options

The following functions can be assigned to [P1] [P2] [P3] programmable function keys. These are located on the bottom left of the radio. Each button has two actions (a short and long press) giving a total of six button assignments. Select correct feature using scrolling bar on programmable key 1 – 3.

There are 13 programmable features that can be assigned to the [P1] [P2] [P3] buttons. (See overleaf).

NOTE: When selecting the monitor button and all of the remaining five buttons are still required, use monitor with long press as setting monitor as short press will blank out the long press option.

Power 	To toggle the transmit Output power between High and Low

Light 	Press to toggle the Auto mode, Light Off or Light On mode.

Monitor	Defeats the squelch operation and un-mutes the speaker on the selected channel.

Lock 	This function can disable all keys except PTT and Emergency Key.

Scan	Starts and cancels scanning operation.

Public Address	When external speaker is attached the radio will output voice audio.

Open/Close	Toggles the radio status between Open or Close mode.

Selcall	Key to enter the Selcall selection mode.

SMS	Key to enter the SMS selection mode.

1200/2400	Toggles the SMS baud rate1200 bps or 2400 bps

DTMF	To enter DTMF selection mode.

Group 	Key to enter the group selection mode.

Asic 	The Asic key is only available on programmable Key 3 long Key. The Asic mode changes the radios default Set-up so should only be done by an authorized dealer. For more details please see operation manual.

Options

Settings and Timings

Settings and Timings

The following features may be selected with full explanation on the next page:

| Parameter Group | Parameter | Selection | Note |
| --- | --- | --- | --- |
| PTT Timer | PTT Timer | Enable/Disable | Switches timer functions On/Off |
| PTT Timer | TX Timer | 10 – 990 sec | Limits TX Time |
| PTT Timer | TX Inhibit | 5 - 75 sec | Inhibits TX for specified time after TX timer |
| PTT Timer | TX Inhibit alert | Enable/Disable | TX inhibit alert |
| PTT Lock | TX Delay | Off or On | Eliminates Squelch tailing noise |
| PTT Lock | Open Lock | Off / Auto Open / LockPTT | Off allows PTT to operate regardless of whether the radio is in an open state (monitor open) or not, Auto Open ‘Auto Open’ opens the monitor of the radio after pressing PTT. Lock PTT ‘Lock PTT’ prevents the keying of PTT unless the radio is open |
| PTT Lock | Busy Lockout | Enable/Disable | Prevents TX when channel is busy. |
| PTT Lock | Mark Idle | Enable/Disable | Busy Channel Lockout is on and carrier is detected, the radio shall be permitted to transmit with valid SAT |
| Scan Setting | Scan TX (without Priority programmed). | Normal Scan TX / RX only no TX | See scanning Parameters |
| Scan Setting | Scan TX (with priority programmed). | Priority scan TX / Priority only TX | See scanning Parameters |
| Scan Setting | Scan Wait | 0 – 15 sec | Delay before scanning starts |
| Scan Setting | Scan Speed Time | 0.5 – 2 sec | Time for one complete scan |
| Scan Setting | Priority LB Time | 0 – 7 sec | Settings and Timings Time for look back |
| Radio Parameter | TX Delay | Enable/Disable | Eliminates squelch tail at end of Transmission |
| Radio Parameter | Monitor | Enable/Disable | The monitor button overrides the squelch circuit |
| Radio Parameter | Hook | Enable/Disable | Changes the action of CTCSS depending on grounding status of the microphone |
| Radio Parameter | Hook Scan | On or Off | Changes the action of scanning depending on grounding status of the microphone |
| Radio Parameter | Beep | Enable/Disable | Gives audible alert tones |
| Radio Parameter | Public Address | Enable/Disable | Passes audio to external Accessory Socket |
| Radio Parameter | Power up Alert | Enable/Disable | Gives audible alert tones |
| Radio Parameter | Power up | Open / Close | Open or closed on switch on- Selcall |
| Radio Parameter | Auto mute | Enable/Disable | Automatically closes the squelch after given time |
| Radio Parameter | Auto mute | 0 – 250 sec | RX Off Time |

## (Fig.18)

## PTT Timer

The PTT timer is the time limit on transmission.
Enable/Disable

## TX Timer

The TX Timeout feature, when enabled, limits the amount of time that a user can continuously transmit.

The time can be set in increments of 10 seconds from 10 to 990 seconds.

## TX Inhibit

The TX Inhibit time is the time that the PTT remains inactive after the TX Timeout has occurred.  Range is 5 - 75 seconds in 5-second increments.

## TX Inhibit Alert

Audible warnings are sounded if the PTT is pressed during the penalty time period.

## Open Lock

Settings and Timings

The radio can control the PTT action in three ways according to the radio busy status.

- Off allows PTT to operate regardless of whether the radio is in an open state (monitor open) or not, and does not change the state of the radio.

- Auto Open ‘Auto Open’ opens the monitor of the radio after pressing PTT.

- Lock PTT ‘Lock PTT’ prevents the keying of PTT unless the radio is open.

## Busy Lockout

Disables the transmitter and generates an alert if user tries to transmit while the channel is busy. If set to ‘mark idle’ then the radio will transmit when a correct SAT is present.

## Mark Idle

- Marked Idle enabled: Can only be enabled if Busy Channel Lockout is ON.  If the Busy Channel Lockout is on and carrier is detected, the radio shall be permitted to transmit if the RX squelch option is valid.

- Marked Idle disabled: Eliminates Marked Idle and defaults back to ‘Busy Channel Lockout’.

## TX Delay

Eliminates squelch tails for SAT transmissions by removing the SAT tone for approximately 200mS before the end of the transmission.

## Monitor

Enables / Disables the monitor button. The monitor button overrides the squelch circuit and any SAT to allow monitoring of the channel.

## Hook

With the hook switch enabled and the microphone grounded CTCSS acts normally. With the hook switch enabled and the microphone not grounded CTCSS is defeated.

## Hook Scan

Settings and Timings

Hook must first be enabled, select hook scan on, and scanning will automatically start when microphone is grounded. Select hook scan off and scanning acts normally.

## Beep

Selects if audio tones are sounded when buttons are pressed, an incorrect operation is performed. If switched off, the LED still continues to flash as necessary.

## Public Address

When selected and external speaker is attached, the radio will output voice audio over the external speaker. The radio does not TX a carrier in this mode. P/A will be shown in display when selected. When active allows operator to enable an external speaker and transmit microphone audio only.

## Power up Alert

Power up alerts are audible tones informing the user the power has been initiated.

## Power up

The radio can be programmed to be either open or closed on power up. This only refers to the selcall state and not squelch.

- Closed: Radio is in muted state.

- Open:  Radio is in unmuted state.

## Auto Mute

Enabled the radio automatically returns to its quiet state after a period of inactivity (see auto-mute timer below). Disabled the radio remains in its open mode until it is manually put into its quiet state (depending on programming).

## Auto Mute Timer

This is the length of time of inactivity after which the radio returns to its quiet state (muted).
0 – 250 sec.

# Scanning Parameters

Scanning Parameters

Scanning Examples

Select scan list by clicking the radio tab and double clicking correct channels in the scan CH box

Open the option tab and select the required programmable key, select scan using scroll bar.

With normal scan TX selected, Scan wait time 4 sec. Scan speed 0.50sec.

On pressing scan button, radio will traverse scan list. If correct carrier and CTCSS/DCS are detected, the radio will stop on channel. The user is not permitted to transmit while scanning (INHIBIT will be displayed), only on a stopped channel is PTT allowed. Scan edit can only be used on a stopped channel. Using the ▼ once scanned channel has locked will temporarily delete that channel from the scan list.

With RX only/noTX selected Scan wait time 4 sec. Scan speed 0.50sec.

On pressing scan button, radio will traverse scan list. If correct carrier and CTCSS/DCS are detected radio will stop on channel. The user is not permitted to transmit at any time (RX ONLY is displayed), Scan edit can only be used on a stopped channel. Using the ▼ once scanned channel has locked it will temporarily delete that channel from the scan list.

Select scan list by clicking the radio tab and double clicking correct channels in the scan CH box. Select the correct priority channel and double click

Open the option tab and select the required programmable key [P1] [P2] [P3], select scan using scroll bar.

With Priority Only TX selected, Scan wait time 4 sec. Scan speed 0.50sec. Priority LB Time 1 sec.

On pressing scan button, radio will traverse scan list, after each channel is scanned the radio will operate as follows: P1=Priority 1, S1=Scan channel P1→S1→P1→S2→P1→S3→P1→S4→P1.

Scanning Parameters

Transmit is allowed on priority channel only, in any state. Scan edit can only be used on a stopped channel. Using the ▼ once scanned channel has locked will temporarily delete that channel from the scan list.

With Priority Scan TX selected, Scan wait time 4 sec. Scan speed 0.50sec. Priority LB Time 1 sec.

On pressing scan button, radio will traverse scan list, after each channel is scanned the radio will operate as follows: P1=Priority 1, S1=Scan channel P1→S1→P1→S2→P1→S3→P1→S4→P1. Transmit is allowed on a stopped channel for a set time period the channel will then change to the priority where transmit is again permitted.

## Priority Look Back Scan

One channel can be programmed by the dealer to be the Priority channel, which will enable ‘Look-back’.  This mode of operation can be used outside of the normal scanning mode. Pressing the Scan button shall activate Priority Look-back. The Priority Look-back causes the radio to periodically ‘Look-back’ to the priority channel for the presence of a carrier regardless of the channel that the user may be on. The frequency that the radio will ‘Look-back’ to the priority channel can be programmed between 1 to 7 seconds in 1-second increments. When carrier, or carrier and tone, are removed, the radio will revert back to the previously selected channel.

Scanning Parameters

Scan Channel Delete

Pressing the “▼” (when in scan mode and stopped on the channel) shall temporarily delete the channel from the scan list. This shall remove that channel from the scan list until the channel is changed or the radio’s power is reset. When power is restored or the scan list channel position is selected again, the originally programmed scan list shall be activated.

## Group Scan Edit Mode

You can edit your pre-programmed Group Scan List by adding or deleting scan list from the Group Scan List. To activate scan list editing, press and hold the P1 button on the front of the radio and turn volume on the radio. Upon entering the scan list edit function, the LCD displays the “Scan Edit” message. To exit the scan list edit function, turn volume off the radio.

1. Select the Scan Group

Each Scan Group would be displayed as “xx_nnn_ xx” which means group, and nnn means channel. To activate scan list editing for selected group, press the P1 button. You can change the Scan Group Number by pressing the ▲▼ buttons.

2. Adding or Deleting to Scan List

If each channel is in the ‘Scan Editable List’, Channel Number would display Scan Icon, to add or delete each channel to Scan Editable List, use P2 button.

## Priority Scan Channel Edit

Priority Channel can be setup by PC programmer. Priority Scan List is also editable by the radio. It is called ‘Priority Channel Edit Mode’. If turn on the radio with pressing P2 Button, the radio enters Priority Channel Edit Mode. In the Priority Edit Mode, the LCD displays the “PscanEdit” message.

1. Select the Priority Group

Scanning Parameters

Each Scan Group dispalys “xx_nnn_xx” means the current Group and ‘nnn’ means channel. To activate P scan list editing for selected Priority group, press the P1 button. You can change the Priority Group Number by up or down button.

2. Adding or Deleting to P Scan List

If each channel is included in P Scan Editable List, Channel Number would display “Pscan” Icon. To add or delete channel from P Scan Editable List, use P2 button.

# Selcall Tones and Sequences

Selcall Tones and Sequences

(Fig.18)

## Tone Sets

There are a number of internationally used Tone Sets. A Tone Set defines the audio frequencies representing each decimal (0 to 9) digit. Other tone frequencies that are not assigned to a decimal digit are also defined and these have special functions.  The available Tone Sets are listed below:

| Tonesets Tone No | EEA | ZVEI1 | ZVEI2 | ZVEI3/ DZVEI | CCIR | PZVEI | CCITT | USER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1981 | 2400 | 2400 | 2200 | 1981 | 2400 | 400 | U/D |
| 1 | 1124 | 1060 | 1060 | 970 | 1124 | 1060 | 697 | U/D |
| 2 | 1197 | 1160 | 1160 | 1060 | 1197 | 1160 | 770 | U/D |
| 3 | 1275 | 1270 | 1270 | 1160 | 1275 | 1270 | 852 | U/D |
| 4 | 1358 | 1400 | 1400 | 1270 | 1358 | 1400 | 941 | U/D |
| 5 | 1446 | 1530 | 1530 | 1400 | 1446 | 1530 | 1209 | U/D |
| 6 | 1540 | 1670 | 1670 | 1530 | 1540 | 1670 | 1335 | U/D |
| 7 | 1640 | 1830 | 1830 | 1670 | 1640 | 1830 | 1477 | U/D |
| 8 | 1747 | 2000 | 2000 | 1830 | 1747 | 2000 | 1633 | Selcall Tones and Sequences U/D |
| 9 | 1860 | 2200 | 2200 | 2000 | 1860 | 2200 | 1800 | U/D |
| A | 1055 | 2800 | 885 | 825 | 2400 | 970 | 0 | U/D |
| B | 930 | 810 | 810 | 740 | 930 | 810 | 0 | U/D |
| C | 2247 | 970 | 740 | 2600 | 2247 | 2800 | 0 | U/D |
| D | 991 | 885 | 680 | 885 | 991 | 885 | 0 | U/D |
| E | 2110 | 2600 | 970 | 2400 | 2110 | 2600 | 0 | U/D |
| Tone Period (ms) | 40 | 70 | 70 | 70 | 100 | 100 | 100 | U/D |

(Fig.19)

**NOTE – U/D = User Definable.  The User column is for the User to enter his own tone frequencies.

## Sequence

This section enables the programming of the different 5 tone analogue selcall sequences to be encoded by the “PM200” (ANI, emergency, group, call 1 – call 3).   The letter “K” represents a number entered via the keypad, “S” represents status digit(s), “X” represents digits from the most recent decode. Up to 8 sequences can be entered (Seq 1 to Seq 8). These are referred to later on in the programmer with regards “Encode”.  When “K” is used, the number of variable digits needs to be entered under ID digits.

## Tone Length (ms)

The duration of the second and successive tones of the selcall sequence set for each of the 8 sequences. Time 0 - 1275

## Lead in (ms)

This is the time delay between activating the transmitter and the transmission of the selcall string.

## 1st Tone (ms)

This is the duration of the first tone of the selcall sequence.  To allow maximum flexibility the tone length does not change when the tone set is changed.

## Lead Out (ms)

This is the time delay between the end of the selcall transmission and the de-activation of the transmitter.

# Encode

(Fig.20)

## Emergency Sequence

Selcall Encode

This is where the sequence or set of sequences to be transmitted as the Emergency Call can be entered.  This refers back to the “Tones and Sequences”, “Encode Sequences” section.  Up to 3 sequences from 1-8 may be transmitted in series e.g. 26 might be entered here.  This will transmit sequence 2 and sequence 6, as a 2 / 5 Tone Emergency sequence.

## Check Open

Two options Enable/Disable.  If set to Enable, Emergency Call can only be sent on an open channel, if set to No, an Emergency Call can be sent at anytime.

## Check Busy

Two options Enable/Disable.  If set to Enable, Emergency Call can only be sent on an unoccupied channel, if set to No, an Emergency Call can be sent at anytime.

## Encode Alert

Two options Enable/Disable.  If set to Enable the radio emits an audible alert to warn the user that an emergency has been transmitted. If set to Disable, no alert is given.

## Repeats

This sets the number of times that the radio will repeat the emergency call.  Ranging from OFF to Permanent.

## Cycle Repeat Time

This is the time delay between re-transmissions of the Emergency sequence.

## Transmit Timer

This is the time that the radio’s microphone is active during the emergency sequence. This time is part of the total repeat timer and must not exceed the cycle repeat time.

## ANI

The ANI sequence is transmitted with respect to the depression of the PTT button.  ANI can be sent either at the Start of TX or End of TX.

## ANI Sequence

This is where the sequence or set of sequences to be transmitted as the ANI can be entered.  This refers back to the “Tones and Sequences”, “Encode Sequences” section.  Up to 3 sequences from 1-8 may be transmitted in series e.g. 26 might be entered here.  This will transmit sequence 2 and sequence 6, as a 2 / 5 Tone ANI sequence. If no ANI is required then this must be left blank.

## ANI at

Selcall Encode

This is where it is defined when the ANI sequence is transmitted.  3 options are available to choose from - OFF, START, and END.

## Multi ANI Delay

This is the time delay between re-transmissions of the ANI sequence.

## Call 1 / Call 2 / Call 3 Sequence

This is where the sequence or set of sequences to be transmitted as Call 1 / Call 2 can be entered.  This refers back to the “Tones and Sequences”, “Encode Sequences” section.  Up to 3 sequences from 1-8 may be transmitted in series e.g. 26 might be entered here.  This will transmit sequence 2 and sequence 6, as a 2 / 5 Tone Call sequence.  Varying the length of the first tone of a second sequence will have the same effect as providing an intersequence gap.

## Check Open

Two options Enable/Disable.  If set to Enable, a Call (1 or 2) can only be sent on an open channel, if set to No, a Call (1,2 or 3) can be sent at anytime.

## Check Busy

Two options Enable/Disable.  If set to Enable, a Call (1 or 2) can only be sent on an unoccupied channel, if set to No, a Call (1 or 2) can be sent at anytime.

## Encode Alert

Two options Enable/Disable.  If set to Enable the radio emits an audible alert to warn the user that a Call has been transmitted. If set to Disable, no alert is given.

## Ack. 1 to Ack. 4

### Sequence

This is where the sequence or set of sequences to be transmitted as an Acknowledgement (Ack. 1-3) can be entered.  This refers back to the “Tones and Sequences”, “Encode Sequences” section.  Up to 3 sequences from 1-8 may be transmitted in series e.g. 26 might be entered here.  This will transmit sequence 2 and sequence 6, as a 2 / 6 ACK sequence.

If ‘X’ is included in the ACK sequence, the radio can be set up to call back the first radio (especially useful in a ‘closed’ system).  This can be achieved by including the users own identity in the transmitted address but the acknowledge sequence will only include the receive address of first radio.

Selcall Encode

# Decoder

## (Fig.20)

## Call 1 / Call 2 / Call 3

Call 1 Address, Call 2 Address, and Call 3 Address

Selcall Decode

This selects the selcall string to be decoded. Up to 8 digits from 0-9, and A. B. C. D. E. The tone set will be that of the encode.

## Control 1 to 4

Ctrl 1 Address, Ctrl 2 Address, Ctrl 3 Address and Ctrl 4 Address

This selects the selcall string to be decoded. Up to 8 digits from 0-9, and A. B. C. D. E. The tone set will be that of the encode.

## Action

When the correct selcall is detected the action is enabled. There are seven states associated with the Action.

| Action |  | Description |
| --- | --- | --- |
| None |  | No action is performed with correct Five tone |
| Close |  | Radio is muted with correct Five tone |
| Open |  | Radio is un-muted with correct Five tone |
| Stun |  | Radio is stunned with correct Five tone |
| Revive |  | Radio is revived with correct Five tone |
| Covert on |  | Radio is put into covert mode with correct Five tone |
| Covert off |  | Radio is taken out of covert mode with correct Five tone |

## (Fig.21)

Close 	Remote muting of the speaker.

Open 	Remote un-muting of the speaker.

Stun	Prevents any transmission from the radio and will also mute the speaker.

Revive 	Re-activates the radio.

Selcall Decode

Covert On 	Valid Secall causes the radio to cycle between TX mode and RX mode. The time periods spent in TX and RX modes are programmed into the addressee radio, however, it will be the Base Station or Selcall Sender that remotely turns this feature ON or OFF.

Covert Off 	Remotely turns the radio covert mode off, i.e. the radio will return back to passive receive mode.

## Acknowledge

This selects the acknowledge string or strings to be transmitted after receiving Call 1 / Call 2 / Call 3.  Please refer to the encode page Ack 1 – 3.

## Selcall Text

When the correct sequence is received text is displayed. Up to 8 characters / numbers.

## Group Call

Specifically used for group calling i.e. the same decoder address sequences across a group.

(Please see action of call addresses, as the same actions apply to the group call).

## Decoder Tone Length

This is the expected length of incoming tones to decode.  This length must be the same as the longest tone length as defined in section.

## Covert Timer

The covert feature enables the remote activation of the transmitter/receiver by another operator via the reception of a valid selcall. This mode is switched on and off by selcall.

## TX Timer

This is the length of time that the radio’s microphone is active.

## RX Timer

This is the length of time that the radio’s receiver is active.

Selcall Decode

# DTMF and Selcall Status

## Selcall Status

There are 9 Selcall decode Status addresses, 8 characters can be used Alpha/Numeric. Please note on the encoding radio the last two digits of the relevant tone set sequence should be set to SS. e.g. 123SS. The last two digits are variable and relate to the status numbers e.g. 01 – 10.

Selcall Status is located on the DTMF and Selcall Status tab.

DTMF and Selcall Status

(Fig.22)

| DTMF Tone Frequencies |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1209hz | 1336hz | 1477hz | 1633hz |
| 697hz | 1 | 2 | 3 | A |
| 770hz | 4 | 5 | 6 | B |
| 852hz | 7 | 8 | 9 | C |
| 941hz | * | 0 | # | D |

## (Fig.23)

## DTMF

Short for Dual Tone Multi-Frequency, the system used by touch-tone telephones. DTMF assigns a specific frequency (consisting of two separate tones) to each number so that it can be easily identified by a microprocessor.

Select enable / disable making sure that one of the programmable keys has been assigned to DTMF on the options page.

## DTMF Lead in Time

This is the time that the radio’s transmitter is active before the first DTMF digit is transmitted (recommended 300ms).

## DTMF Lead out Time

DTMF Lead out time is the amount of time the transmitter is on after the DTMF is sent 0 – 1000 ms

Mark Time

DTMF Mark Time

DTMF and Selcall Status

The mark time is the amount of time = Positive slope

0 – 100ms

Space Time

DTMF Space Time

The Space time is the amount of time = Negative slope

0 – 100ms

## DTMF Address

This is where the sequence or set of sequences to be transmitted as DTMF 1 ~ 9 can be entered. Up to 15 Characters from 0-9, a -d, *, # may be transmitted.

# Modem SMS

## (Fig.24)

## SMS

The SMS (short message service) is a pre-programmed short message that can be sent over air. Check that one of the programmable keys has been assigned to SMS on the options page. Please note using a Keypad microphone real time messaging can be achieved.

Modem SMS

Select the correct baud rate by programming one of the programmable keys.

## TX On Delay

Time delay between when the transmitter is enabled and modulated data signal is sent.

## TX Off Delay

Gap between when modulated signal has finished and transmitter switches off.

Contact Details

Contact Details

Should you have any queries regarding this manual, or the information within it, please contact:

Maxon House,

Cleveland Road,

Hemel Hempstead,

Hertfordshire,

United Kingdom,

HP2 7EY

Tel: + 44 (0) 1442 267 777

Fax: + 44 (0) 1442 215 515

info@maxoncic.co.uk

www.maxoncic.co.uk

Technical Support:

Tel: + 44 (0) 1442 226 170

Email: technicalsupport@maxoncic.co.uk

Customer and Sales Support:

Telephone : + 44 (0) 1442 267 777

Email: sales@maxoncic.co.uk

Repairs:

Tel: + 44 (0) 1442 267 777

Email: repairs@maxoncic.co.uk