HVN9128A

Version R08.01.00 of the GP300/GP350 RSS now supports the new

GP350 models.  PEOPLE FINDER and RF LINKS are also supported.

P110 models are not supported in this release.

Please continue to use version R07.00.00 of the GP300 RSS for

the P110 models.

These GP350 radio models are supported:

P93MGC00A1__        P94MGC00C1__

P93MGC00A2__        P94MGC00C2__

P93MGC00C1__        P94MGC00C3__

P93MGC00C2__        P94MGC00C4__

P93MGC20A1__        P94MGC20A2__

P93MGC20A2__        P94MGC20C1__

P93MGC20C1__        P94MGC20C2__

P93MGC20C2__        P94MGC20C3__

P94MGC00A2__        P94MGC20C4__

These GP300 radio models are supported:

P93YPC00C2__        P93YPC20G2__       P94YPC20A2__

P93YPC00C3__        P94YPC00A1__       P94YPC20A3__

P93YPC00C4__        P94YPC00A2__       P94YPC20A4__

P93YPC00C8AA        P94YPC00A3__       P94YPC20C1__

P93YPC00D1__        P94YPC00A4__       P94YPC20C2__

P93YPC00D2__        P94YPC00C1__       P94YPC20C3__

P93YPC00D3__        P94YPC00C2__       P94YPC20C4__

P93YPC00D4__        P94YPC00C3__       P94YPC20C5__

P93YPC20A1__        P94YPC00C3__       P94YPC20C6__

P93YPC20A2__        P94YPC00C4__       P94YPC20D1__

P93YPC20C1__        P94YPC00C5__       P94YPC20D2__

P93YPC20C2_F        P94YPC00D1__       P94YPC20D3__

P93YPC20C2__        P94YPC00D2__       P94YPC20D4__

P93YPC20C8AA        P94YPC00D3__       P94YPC20D5__

P93YPC20D1__        P94YPC00D4__       P94YPC20D6__

P93YPC20D2_F        P94YPC00D5__       P94YPC20G2__

P93YPC20D2__        P94YPC20A1__

The  MAIN  Menu is the top level of the program from which you  select  the

type of function that you wish to perform.  All selections are made via the

"Function  Keys", labeled F1 thru F10, on your keyboard.  Press the  ESCape

key to return to the MAIN Menu from any where in the program.

After  making a selection, you will be directed to similar menus and  /  or

data  entry screens for RF alignment or codeplug data.  You must  initially

load  data from a radio (or  disk) via the F3 GET/SAVE function before  you

will be permitted to CHANGE or VIEW any codeplug data.

Call   Radius   Product Services 800-356-1520 in  Mt.   Pleasant, Iowa  for

problems  not covered in your Radio Service Software User's manual or  your

radio Service manual.

.P

MAIN MENU (cont)

Function Key Descriptions:

F1 - Additional  information  is available for every menu via the  F1  HELP

key.    Specific  help is also available for each field  on  all  data

entry  screens.  Generic help is available within any HELP screen  via

the MORE HELP function.

F2 - SERVICE  is a multi-level menu that permits access to radio  ALIGNMENT

and   PARTS  REPLACEMENT   screens.   Radius   contains  no   internal

alignment  or tuning adjustments; all alignment is performed  via  the

SERVICE screens.

.P

MAIN MENU (cont)

A radio must be connected to your computer via the RIB before you will

be  permitted  to  access the SERVICE screens.   All  SERVICE  screens

access  the codeplug directly; therefore, you do NOT have to read  the

codeplug  data (via the GET/SAVE functions) before using  the  SERVICE

screens.

F3 - GET/SAVE is used to read codeplug data from a radio and/or retrieve an

archived  codeplug from a diskette or hard disk for  editing  purposes

(via  the  CHANGE/VIEW function).  GET/SAVE is also  used  to  program

edited codeplug data back into the radio or to create an archive  file

on a diskette (or hard disk).

.P

MAIN MENU (cont)

F4 - CHANGE/VIEW  is  a multi-level menu that is used to change,  view,  or

modify  codeplug  features and option  configurations.     All  Radius

codeplug  parameters  are classified as either RADIO-WIDE  or  CHANNEL

related.  CHANGE/VIEW permits access to each of these categories.

Unlike  the  SERVICE  function, a codeplug must be  loaded  into  your

computer's  memory (via the GET/SAVE functions) before you can  access

the CHANGE/VIEW screens.  You may CHANGE/VIEW an archive file  without

a radio connected.

.P

MAIN MENU (cont)

F5 - A PRINT utility is available to produce permanent records of   codeplug

configurations  and/or RF alignment settings.  A printer  is   required

and should be connected to your computer per your instruction manual.

F6 - A FILE MAINTENANCE utility is available to create sub-directory  paths,

delete old archive files, restore archive files, and create new back-up

files.

F7 - Not Used.

F8 - Not Used.

.P

MAIN MENU (cont)

F9 - SETUP  is  used to configure your  Radius  Radio Service  Software  to

your  particular  application.   Default  disk  drives,  communication

ports,  and  even  screen colors may be customized  to  your  specific

needs.

F10 - Select  EXIT  to quit the program and return to  DOS.   Be  sure  all

desired codeplug changes have been programmed back to the radio,  and

that  an archive copy has been made.   Otherwise all changes will  be

lost  since  returning  to DOS erases this data from  the  computer's

memory.

.P

KEYBOARD OPERATION

TAB (or ENTER):         Advance Cursor to Next Data Field

Shift TAB:              Backup Cursor to Last Data Field

UP / DOWN Arrow Keys:   Increment / Decrement Value or Selection

LEFT / RIGHT Arrow:     Move Cursor Within Data Field

INSERT:                 Insert Space at Current Cursor Position

BACKSPACE:              Erase Data Within Field & Move Cursor Left

DELETE:                 Erase Current Character

PAGE UP / PAGE DN:      Scroll Displayed Data UP or DOWN

HOME:                   Move Cursor To Upper Left Data Field

F1:                     Additional Information

F2 - F8:                Execute Labeled Function

F10:                    Return to the PREVIOUS Menu

ESC:                    Return to the MAIN Menu

.P

MOTOROLA Radio Service Software             Radius  GP300/GP350

Part # (3 1/2).......HVN9128A               Model #........

Serial #.......

Software Ver...

Software Version.....

Date.................                       Band...........

Manual #.............6880904Z09 (GP300/GP350)

RIB Part #...........01-80353A74

HLN9214A

.P

CABLE Part #'s

PC/XT -> RIB.......30-80369B71

HKN9215A

AT    -> RIB.......30-80369B72

HKN9216A

RIB   -> Radio.....30-80070N01

HKN9857A

.P

SERVICE                                                                2000

All radio alignment and parts replacement procedures are accessed from  the

SERVICE  menu.   A radio must be connected to your computer via a  RIB  and

cables  and the radio turned on before you will be permitted to access  the

SERVICE screens.

!!!  WARNING  !!!

Do  NOT switch radios in the middle of any SERVICE procedure.   Always  use

the EXIT key to return to the previous menu screen before disconnecting the

radio.

Call  Radius  Product Services  (800) 356-1520 in Mt.  Pleasant,  Iowa  for

problems  not covered in your Radio Service Software User's manual or  your

radio Service manual.

.P

SERVICE (cont)

All  SERVICE screens read and program the radio codeplug directly;  you  do

NOT  have to use the GET/SAVE functions unless you are editing or  printing

data.   You will be prompted at each SERVICE screen to save the new  values

before exiting the screen.

Refer  to your  Radius  Radio Service Software User's Manual  for  detailed

service procedures.

.P

SERVICE (cont)

Function Key Descriptions:

F2 - The ALIGNMENT function is used to perform standard radio alignment  on

Level Set Deviation, Transmitter  VCO Deviation,  Reference Oscillator

Warp, and  Transmitter Power Set.

F6 - The  BOARD REPLACEMENT function is used for servicing the  radio  when

board   repairs   and/or  replacement  are   required.    Step-by-step

instructions are given for all re-alignment procedures.

.P

ALIGNMENT                                                              2200

Refer  to your  Radius  Radio Service Software User's Manual for  ALIGNMENT

procedures.

Standard  periodic  alignment procedures are performed from  the  ALIGNMENT

Menu.  These include:

Level Set Adjustment

Transmitter VCO Deviation Adjustment

Reference Oscillator Warp Adjustment

Transmitter Power Set

.P

LEVEL SET                                                              2220

This  alignment  screen allow the  values for  Maximum  Volume,  Microphone

Gain, System Deviation and PL/DPL Deviation to be set.

Maximum Volume sets the peak volume range for the volume control.

Microphone Gain can be adjusted to four levels of sensitivity.

System Deviation adjusts the total modulation of the carrier frequency.

PL/DPL deviation can be varied over a narrow range.

The  System Deviation and the PL Deviation CANNOT be cloned when using  the

Smart Cloning Adapter or the cloning function key.

.P

DEVIATION                                                              2230

Refer  to  your   Radius   Radio Service Software  User's  Manual  for  the

DEVIATION ADJUSTMENT procedure.

Transmitter  Deviation is increased or decreased by first keying the  radio

via  F6,  and  then by pressing the UP/DOWN  arrow  keys  respectively.   A

relative  deviation  value will be displayed, but  the  actual  transmitter

deviation  must  be determined from your service monitor.  The  radio  will

transmit  on  the test frequencies displayed on the screen  and  should  be

terminated into a 50 ohm load or service monitor.

Using  the UP/DOWN arrow keys, adjust Tx Deviation per your User's  Manual.

Press F6 again to de-key the radio.

.P

REFERENCE OSCILLATOR WARP                                              2250

Refer  to  your   Radius   Radio Service Software  User's  Manual  for  the

REFERENCE OSCILLATOR WARP ADJUSTMENT procedure.

The  Reference Oscillator is warped by first keying the radio via  F6,  and

then  by  pressing the UP/DOWN arrow keys respectively.   A  relative  warp

position  will be displayed, but the actual transmitter frequency  must  be

determined from your frequency counter or service monitor.  The radio  will

transmit  on  the  test frequency displayed on the  screen  and  should  be

terminated into a 50 ohm load or service monitor.

Using  the UP/DOWN arrow keys, adjust the Reference Oscillator Warp to  the

displayed frequency.  Press F6 again to de-key the radio.

.P

TRANSMITTER POWER                                                      2270

Refer  to  your   Radius   Radio Service Software  User's  Manual  for  the

TRANSMITTER POWER ADJUSTMENT procedure.

Transmitter Power is adjusted by first keying the radio via F6, and then by

pressing the UP/DOWN arrow keys to increase or decrease power respectively.

A  relative Tx Power value (not watts!) will be displayed, but  the  actual

transmitter power output must be determined from your service monitor.  The

radio  will  transmit  on the test frequency displayed on  the  screen  and

should be terminated into a 50 ohm load or service monitor.

Using the UP/DOWN arrow keys, adjust the Transmitter Power per your  User's

Manual.   Press F6 again to de-key the radio.

.P

SQUELCH                                                                2290

This aligment screen adjusts the carrier squelch opening threshold  for all

channels of the radio.   Lower values indicate looser squelch, while higher

numbers tighten the squelch.   The radio will automatically  be placed into

CSQ operation  to perform  this adjustment.   The current  frequency wil be

displayed.

.P

PARTS REPLACEMENT/BLANK BOARD INITIALIZATION                           2600

Refer to your  Radius  Service Manual for PARTS REPLACEMENT or BLANK BOARD

INITIALIZATION procedures.

Calibration is required after the following parts replacement:

Main Board

Power Amplifier

Squelch Adders

!!!  WARNING  !!!

Do NOT turn off the radio or disconnect it from the computer while

attempting to PROGRAM the radio.  Interrupting the programming process

WILL destroy the codeplug contents and completely DISABLE the radio!

PARTS REPLACEMENT/BLANK BOARD INITIALIZATION (Cont)

The PROGRAM RADIO function on the BLANK BOARD INITIALIZATION screen is used

to initialize the codeplug in a blank board to a codeplug for the selected

model.  This function will initialize the blank board with the serial

number entered in the serial number field on this screen.  This function

will be disabled if the serial number to be programmed into the blank board

is not a proper serial number.

.P

PRODUCT LINE                                                           2611

This is the name of the Product Line the blank board supports.  You can

only initialize the blank board to models within the supported product

line.  This field is defaulted to the supported Product Line.  If the

blank board supports more than one Product Line, use the UP/DOWN arrow

keys to select the product line.

.P

MODEL NAME                                                             2612

This is the name of the Model the blank board supports.  You can only

initialize the blank board to supported models.  This field is defaulted to

the supported model.  If the blank board supports more than one model, use

the UP/DOWN arrow keys to select the model name.

.P

RANGE                                                                  2613

This is the frequency Range of the model the blank board supports.  The

Range must be a valid range for the selected model.  This field is

defaulted to the Range of the supported model.  If the blank board supports

more than one Range for the model, use the UP/DOWN arrow keys to select the

range.

.P

MODEL NUMBER                                                           2614

This is the Model Number the blank board supports.  You can only initialize

the blank board to supported models.  This field is defaulted to the

supported Model.  If the blank board supports more than one model, use

the UP/DOWN arrow keys to select the Model Number.

.P

PANEL NUMBER                                                           2615

Enter the Panel Number of the 'Target' radio.  The PROGRAM RADIO function

will initialize the blank board with the Panel Number entered.

The default value is 000.  The valid range is from 000 to 255.

.P

SERIAL NUMBER                                                          2616

Enter the Serial Number of the `Target' radio.  This number can be found

on the outside casing of the radio.  You will not be able to initialize

the blank board unless you enter a proper Serial Number.  Once the radio is

programmed with this Serial Number, you will not be able to change the

Serial Number.

.P

MAIN BOARD                                                             2620

Refer to your  Radius Service Manual for MAIN BOARD REPLACEMENT procedures.

This  menu is structured for execution of the calibration procedures  in  a

top-to-bottom order to ensure proper calibration.

Steps  7 & 8 are model-dependent and will be by-passed for  non  applicable

models.

!!!  WARNING  !!!

After  Step  8  is completed, the radio codeplug should be  READ  and  then

archived (SAVED) via the GET/SAVE functions.

.P

REFERENCE CRYSTAL DATA                                                 2621

Refer  to your  Radius  Radio Service Software User's Manual  for  entering

the Crystal Coefficient Data.

.P

CALIBRATE Tx POWER                                                     2622

Refer  to  your   Radius   Radio Service Software  User's  Manual  for  the

TRANSMITTER POWER CALIBRATION procedure.

This procedure is used to calibrate the power amplifier.

.P

CALIBRATE Tx DEVIATION                                                 2623

Refer  to  your   Radius   Radio Service Software  User's  Manual  for  the

TRANSMITTER DEVIATION CALIBRATION procedure.

This screen is very similar to the Calibrate Tx Power  screen,  except that

the scale represents the deviation adjustment instead of power.

.P

PL DEVIATION                                                           2624

Refer  to  your  Radius  Radio Service Software User's Manual  for  the  PL

DEVIATION ALIGNMENT procedure.

.P

DPL DEVIATION                                                          2625

Refer  to  your  Radius  Radio Service Software User's Manual for  the  DPL

DEVIATION ALIGNMENT procedure.

!!!  WARNING  !!!

After  Step  8  is completed, the radio codeplug should be  READ  and  then

archived (SAVED) via the GET/SAVE functions.

.P

POWER AMPLIFIER                                                        2640

Refer  to  your  Radius  Service Manual for the POWER AMPLIFIER REPLACEMENT

procedures.

This  menu is structured for execution of the calibration procedures  in  a

top-to-bottom order to ensure proper calibration.

!!!  WARNING  !!!

After  Step  2  is completed, the radio codeplug should be  READ  and  then

archived (SAVED) via the GET/SAVE functions.

.P

SQUELCH ADDERS                                                         2690

Refer to your Radius Service Manual for the  SQUELCH CALIBRATION procedure.

This proceudre  adjusts  the carrier  squelch opening  threshold for  each

channel  position  of the radio.   The adjustment  will  be made  on  each

channel's appropriate frequency.  This allows you to customize the squelch

for interference patterns or personal preference.

The radio will  automatically be placed into CSQ operation to perform this

adjustment.  The current frequency will be displayed.

This information can  be cloned  using  the Smart Cloning  Adapter  or the

cloning function key.

.P

GET/SAVE                                                               3000

The  GET/SAVE functions are used to transfer codeplug data from your  radio

or an archive file into your computer in order for you to Change, View,  or

Print it.  GET/SAVE functions also permit you to PROGRAM modified data back

into your radio or to SAVE a copy of the codeplug data in an archive file.

!!!  WARNING  !!!

Do  NOT  turn  off  the radio or disconnect  it  from  the  computer  while

attempting  to PROGRAM the codeplug.  Interrupting the programming  process

WILL destroy the codeplug contents and completely DISABLE the radio!

.P

GET/SAVE (cont)

Function Key Descriptions:

F2 - The READ CODEPLUG function reads the information (data) stored in  the

radio codeplug (EEPROM) and transfers it to the computer's memory.

A  radio and RIB must be properly connected to the computer and  power

turned on before you attempt the READ function.

The  time  required to READ a codeplug will depend  directly  on  your

computer  type  and  the size of the codeplug you  are  reading.   The

status of the READ operation is displayed at the bottom of the screen.

.P

GET/SAVE (cont)

F3 - The  GET  FILE  function is used to retrieve an archive  file  from  a

diskette  or hard disk to the computer.  Once retrieved, the file  may

be  modified via the CHANGE/VIEW functions or programmed into a  radio

just  as  codeplug  information read from the radio via  the  F2  READ

function.

F5 - The  CLONE  CODEPLUG  function is used to copy personality information

from a master radio  or  archive file into one or more  target radios.

.P

GET/SAVE (cont)

F7 - The  SAVE FILE function is used to create (or update) an archive  copy

of the codeplug information onto a diskette or hard disk.   An archive

copy  of every radio installed or serviced is STRONGLY recommended  in

order to be able to quickly restore customer information is case of  a

codeplug failure.

.P

GET/SAVE (cont)

F8 - The PROGRAM CODEPLUG function is used to transfer codeplug information

from the computer to the radio codeplug.

A  radio and RIB must be properly connected to the computer and  power

turned on before you attempt the PROGRAM function.

The  time required to PROGRAM a codeplug will depend directly on  your

computer  type and the size of the codeplug you are programming.   The

status  of  the PROGRAM operation is displayed at the  bottom  of  the

screen.

.P

GET FILE                                                               3300

The  GET FILE function is used to retrieve an archive file from a  diskette

or hard disk.  Once retrieved, the file may be modified via the CHANGE/VIEW

functions or programmed into a radio just as codeplug information read from

the radio via the F2 READ function.

Use  the TAB (or ENTER) key to select the serial number of the radio to  be

retrieved.    The  corresponding  model  number,   customer  identification

information,  and  the date the file was made will be displayed across  the

middle  of  the screen.    The F8 function key is used to GET the  selected

file as described below.

.P

GET FILE (cont)

Function Key Descriptions:

F2 - The  CHANGE  ARCHIVE function is used to specify  the  directory  path

where the archive file is to be located.  The default archive path

will always be the specified default path from the PC CONFIGURATION

screen  (See MAIN MENU, F9, F3).

F5 - The  PRINT ARCHIVE function is used to print the contents of the

archive.  That is, it will print a list of all files in the archive.

.P

GET FILE (cont)

F6 - The  ENTER S/N (serial number) function permits you to enter  directly

the  serial number of the file you wish to retrieve.   After  entering

the  serial  number,  press  ENTER and the computer  will  search  the

specified  directory path to locate the file for that  serial  number.

The  F8  function  key is used to GET the selected file  as  described

below.

F8 - The  GET SELECTED function is used to GET the archive file highlighted

on the screen.   After the file is retrieved, the computer will return

to the GET/SAVE menu.

.P

ARCHIVE PATH                                                           3301

Enter  the  directory path where the archive file is to  be  located.

The  default archive path will always be the specified default  path   from

the PC CONFIGURATION screen (See MAIN MENU, F9, F3).

Please  refer to your computer owner's manual for a complete description of

directories and path names.

.P

SERIAL NUMBER                                                          3302

Enter  the  serial  number of the codeplug archive file that  you  wish  to

retrieve and press ENTER.  The computer will search the specified directory

path  to  locate the file if it exists.   Only the specified path  will  be

searched.

Once the  file  is located,  press F8 to GET the SELECTED file.   A  Status

message will inform you if the file does not exist on the specified path.

.P

CLONE CODEPLUG                                                         3500

The Clone function copies the serial number and tuning data from an archive

file and overlays this information over the current data in the  computer's

memory.  To use the clone function use the following steps:

1.   Go to the GET/SAVE Screen.

2.   Load the Master radio or archive file into the computer  memory  with

the GET/SAVE screen functions F2 or F3.

3.   Press function F5 and the Clone File screen will be displayed.

4.   Select the desired archive file to be cloned.  Only Radios of the same

model  may  be cloned.  Any attempt to mix models will  result  in  an

error message.

5.   If  necessary  go to the CHANGE/VIEW screens to  make  any  additional

changes.

.P

6.   Save the radio codeplug data by saving as an archive file or saving to

the radio to be cloned using the GET/SAVE screen.

This completes the procedure for cloning a archive file or radio.

.P

SAVE FILE                                                              3700

The SAVE FILE function is used to create (or update) an archive copy of the

codeplug information onto a diskette or hard disk.  If you are creating the

file  on  a hard disk,  you will automatically be instructed to backup  the

data onto a diskette.

An optional 15-character name can be given to an archive file to facilitate

tracking.

You  are  STRONGLY  encouraged  to  make an archive  copy  of  every  radio

installed  or  serviced  in order to be able to  quickly  restore  customer

information is case of a codeplug failure.

.P

SAVE FILE (cont)

Function Key Descriptions:

F2 - The  CHANGE  ARCHIVE function is used to specify  the  directory  path

where  the  archive file is to be located.   The default archive  path

will  always be the specified default path  from the SETUP  Menu  (See

MAIN MENU, F9).

F8 - The  SAVE FILE function executes the save process.   Do NOT  press  F8

until you have entered the customer identification information.

.P

CUSTOMER ID                                                            3701

Enter  an  optional  15-character  (max)  name  for  the  archive  file  to

facilitate tracking purposes.

.P

BACKUP FILE                                                            3710

The BACKUP FILE procedure is used to make a backup copy of any archive file

SAVED on a hard disk.   If you are archiving to a diskette, you will NOT be

prompted  for a backup.

Diskette  backups should be handled via standard DOS file copy  procedures.

Refer to your computer user's manual for additional information.

Function Key Descriptions:

F2 - The CHANGE BACKUP path function is used to specify the directory  path

where the backup file is to be located.   The default backup path will

always  be  the specified default path  from the SETUP Menu (See  MAIN

MENU, F9).

.P

BACKUP FILE (cont)

F8 - The MAKE BACKUP function executes the backup process.  After verifying

that  the Backup Path is correct and the backup diskette is installed,

press F8 to make the backup copy.   You will automatically be returned

to the GET/SAVE menu.

.P

BACKUP PATH                                                            3711

Enter  the  directory path where the backup file is to  be  located.

The default backup path will always be the specified default path  from the

SETUP Menu (See MAIN MENU, F9).

Please  refer to your computer user's manual for a complete description  of

directories and path names.

.P

CHANGE/VIEW                                                            4000

CHANGE/VIEW is a multi-level menu that is used to change,  view,  or modify

codeplug features and option configurations.    All codeplug parameters are

classified  as either RADIO-WIDE or MODE(channel) related.   CHANGE/VIEW

permits access to each of these categories.

Unlike the SERVICE function, a codeplug must be loaded into your computer's

memory  (via the GET/SAVE functions) before you can access the  CHANGE/VIEW

screens.  You may CHANGE/VIEW an archive file without a radio connected.

Warning:  CHANGE/VIEW does NOT actually modify the radio codeplug data, but

instead  it  modifies  a  copy of the  data  retrieved  from  the

codeplug (or archive file) via the GET/SAVE functions.

.P

CHANGE/VIEW (cont)

After  all  CHANGE/VIEW modifications  are  completed,  you  MUST

return to the GET/SAVE menu and PROGRAM the changes back into the

radio  or  SAVE  them  to a  new  archive  file.   Otherwise  the

modifications will be lost when you turn off your computer.

Function Key Descriptions:

F2 - RADIO-WIDE  Configuration is used to CHANGE/VIEW radio parameters  and

options that affect overall radio operation, i.e. those not related to

a specific channel such as the time-out-timer.

F5 - MODE Configuration is used to CHANGE/VIEW channel specific information

such as frequencies, squelch type, squelch codes and scan lists.

.P

CHANGE/VIEW (cont)

Function Key Descriptions:

F6 - PHONE MEMORY Configuration is used to program the Phone Memory Dialer.

(This menu option is ONLY available on phone capable models).

F7 - SIGNAL SYSTEM Configuration is used to CHANGE/VIEW Signalling System

parameters for MDC-1200, Quick Call, DTMF, and DTMF/Phone type

signalling systems.  (This menu option is ONLY available on signalling

capable models).

.P

PRINT                                                                  5000

The  PRINT  function is  used to  produce  permanent  records  of  codeplug

configurations  and/or  RF alignment settings.   A printer is required  and

should  be  connected  to your computer per your  computer  user's  manual.

Graphics capability is NOT required.

You  must use the GET/SAVE functions to first READ or GET a  codeplug   for

printing  radio configuration data (see F2 below);  you must have  a  radio

connected  to the computer for printing an alignment summary (see F5 below).

Each printout contains radio model and serial number information,  software

version numbers,  Radio Service Software version numbers,  and the date and

time of the printout.

.P

PRINT (cont)

Function Key Descriptions:

F2 - RADIO  CONFIGURATION is a multi-level  menu  that  allows the  user to

print either the  Radio Wide Configuration, the  Mode Configuration or

the full Mode Configuration.

F4 - The  PHONE LIST Summary  function  prints the  list of entries  in the

phone  memory dialer  (only available on phone capable models).

F5 - The  ALIGNMENT  Summary  function prints the  current  relative  value

setting for all SERVICE alignment adjustments.

.P

PRINT (cont)

Funtion Key Descriptions:

F6 - The  ARCHIVE  Files function prints a list of all  codeplug  files  by

serial number for a specified directory path.

F7 - The  SIGNALLING LIST SUMMARY function prints a compressed  summary  of

all signalling systems that are programmed in the radio.

.P

PRINT RADIO CONFIGURATION MENU                                         5200

This  PRINT submenu  provides various functions for printing radio wide and

mode configuration information.

F2 - The RADIO WIDE Configuration function  prints all the information from

the radio wide configuration screen.

F4 - The MODE Configuration (Summary) function prints a one page summary of

the current mode configuration.

F5 - The  MODE Configuration (All Parameters) function prints all the info

contained on the mode configuration screen for each mode.

.P

PRINT ARCHIVE FILES                                                    5600

This function will print a  list of the  archive files  which exist in  the

specified archive path.

.P

FILE MAINTENANCE                                                       6000

The  File Maintenance Menu is designed to help you manage your  files.   It

can  be used to restore lost Archive Files from your Back-up Files,  create

new Back-up Files, create directory paths, and delete Archive Files.

.P

CREATE DIRECTORY PATH                                                  6400

This  screen  will  create  any  desired  directory  path  by  adding  sub-

directories that can be accepted by the DOS operating system.

Enter the new directory path and press the Enter key.

An Example directory path would be:

C:\MRSS\GP300\ARCHIVE\LOCAL <Enter>

This   would   add  the  sub-directory  "LOCAL"  under  the   sub-directory

"ARCHIVE".  Only one sub-directory may be added at a time.

.P

DELETE ARCHIVE FILE                                                    6600

The  Delete Archive File screen allows you to delete any Archive File  from

the directory displayed in the Archive Path.

F3 - The CHANGE ARCHIVE function allows you to change the Archive Directory

Path.  Enter the new Path and Press Enter.

F5 - The  PRINT function print the serial numbers of the archived files  to

your printer.

F7 - The  DELETE  FILE function  will delete the file associated  with  the

highlighted  serial number.  You can move the highlight cursor to  the

desired  serial number by using the Tab and Enter keys.  Press  F7  to

delete  the file.  The Computer will ask you to confirm the delete  by

pressing F7 again.

.P

RESTORE ARCHIVE                                                        6700

This screen  is used to  restore lost  Archive Files from a  Back-up  Files

Disk.

F3 - The  CHANGE SOURCE function is used to point to the Back-up  directory

Path.  Enter the new Path and press Enter.

F4 - The  CHANGE TARGET function is used to point to the Archive  directory

where  you  want to store the recovered Archive File.  Enter  the  new

Path and press Enter.

.P

RESTORE ARCHIVE (cont)

F7 - The  RESTORE  FILE  function  is  used  to  move  the  Back-up   File,

highlighted  on the display, to the directory displayed on the  Target

path.  You can move the highlight cursor to the desired serial  number

by using the Tab and Enter keys.

.P

UPDATE BACK-UP FILES                                                   6800

The Update Back-up Files Screen allows you to create new back-up copies  of

your archive files.

F3 - The  CHANGE SOURCE function is used to point to the Archive  directory

Path that you want to back-up.   Enter the new  Path and  press Enter.

F4 - The  CHANGE TARGET function is used to point to the Back-up  directory

where  you  want to store the Back-up Files.  Enter the new  Path  and

press Enter.

F7 - The  CREATE  BACKUP  function  will  copy  the selected  Archive  File

contained in the Source Path to the directory of the Target Path.

.P

SETUP                                                                  9000

The  SETUP  functions are used to configure your Radio Service Software  to

your particular application and computer environment.

Function Key Descriptions:

F3 - PC  CONFIGURATION is used to set default disk drive paths for  Archive

files and Backup files.   It is also used to select which Asynchronous

Communications  Port  (COM 1 or COM 2) the RIB (Radio  Interface  Box)

will be connected to.

Please   refer  to  your  computer  owner's  manual  for  a   complete

description of path names and Asynchronous Communication Ports.

.P

SETUP (cont)

Note:  Your  Radio  Service Software will automatically determine  at  what

clock  speed  your  computer  is  operating.    No   user-selectable

parameters are required.

F7 - SCREEN  COLORS  is  used  to enable the  Color  display  option.   The

standard  default  configuration for Radio Service Software is  for  a

MONOCHROME display monitor.   However, if you have a color system, the

display quality will be greatly enhanced by enabling the color option.

.P

CONFIGURE COMPUTER                                                     9300

PC  CONFIGURATION is used to set default disk drive paths for Archive files

and  Backup  files.    It  is  also  used  to  select  which   Asynchronous

Communications  Port (COM 1 or COM 2) the RIB (Radio Interface Box) will be

connected to.

Please refer to your computer owner's manual for a complete description  of

directory path names and Asynchronous Communication Ports.

.P

CONFIGURE COMPUTER (cont)

Function Key Descriptions:

F3 - COMM TEST is used to verify that your computer is set up properly  and

is able to READ and PROGRAM a radio codeplug properly.

After  your  computer  and RIB are connected per  your  Radio  Service

Software  manual  and you have selected the appropriate  communication

port, turn on your radio and execute COMM TEST by pressing F3.

COMM  TEST will verify your system is functioning properly by  sending

commands  to  the  radio and checking for  the  proper  response.   No

codeplug changes will result from these commands.  An OK response will

be displayed in the Status Window if the system checks OK.

.P

CONFIGURE COMPUTER (cont)

Otherwise,  error  messages will be displayed,  and you should consult

your manual for proper corrective action.

F8 - SAVE  is used to save the configuration information to a file on  your

program  disk.   Every time you use the Radio  Service  Software,  the

configuration  that  you  SAVED last will be  used.   At  anytime  the

configuration may be changed and SAVED.

F9 - RESET  DEFAULT  is  used to reset the parameters on this screen  to  a

default   value.    However,   RESET   DEFAULT  does  NOT   save   the

configuration.   If the default values are desired, you must save them

via the SAVE (F8) function.

.P

ARCHIVE DISK DRIVE PATH NAME                                           9301

Enter the DEFAULT directory path where archive files are to be located.

The GET/SAVE SAVE FILE function will default to this path.

Please  refer to your computer owner's manual for a complete description of

directories and path names.

.P

BACKUP DISK DRIVE PATH NAME                                            9302

Enter the DEFAULT directory path where backup files are to be located.

The  GET/SAVE  BACKUP FILE function will default to this path  when  making

backup  copies  of  codeplug  files archived to a hard  disk.  If  you  are

archiving to a diskette, you will not be prompted for a backup.

Diskette  backups should be handled via standard DOS file copy  procedures.

Refer  to your computer user's manual for additional information  regarding

coping files, directories, and path names.

.P

SERIAL COMMUNICATIONS PORT (Default = COM 1)                           9303

Use  the UP/DOWN arrow keys to select to which Asynchronous  Communications

Port (COM 1 or COM 2) your RIB (Radio Interface Box) is connected.

If  you  are  not sure how your computer is configured or if you  have  two

Asynchronous Communications Ports, first select COM 1 and use the COMM TEST

(F3) function to communicate with a radio.  If the test fails, select COM 2

and  repeat  COMM  TEST.   Be sure that all cables and power  supplies  are

connected  per  the  instructions in your  Radio  Service  Software  User's

manual.

Please refer to your computer owner's manual for a complete description  of

Asynchronous Communication Ports and how to configure them.

.P

CONFIGURE COMPUTER SCREEN                                              9700

SCREEN  COLORS  is used to select the type of display monitor that you  are

using with your computer, i.e. MONOCHROME or COLOR.

For  proper  color  operation,  you  must  have a  color  monitor  and  the

appropriate color display interface card installed in you computer.  Please

refer  to  your computer owner's manual and/or your computer dealer if  you

have questions regarding the color capability of your computer.

You  may  also further customize your screen by selecting  colors  for  the

screen TEXT,  STATUS LINE,  MESSAGE LINE,  TEXT HIGHLIGHT,  BACKGROUND, and

SCREEN OUTLINE.

.P

Function Key Descriptions:

F8 - SAVE  is used to save the configuration information to a file on  your

program  disk.   Every  time you use the Radio Service  Software,  the

configuration  that  you  SAVED last will be  used.   At  anytime  the

configuration may be changed and SAVED.

F9 - RESET  DEFAULT  is  used to reset the parameters on this screen  to  a

default   value.    However,   RESET   DEFAULT  does  NOT   save   the

configuration.   If the default values are desired, you must save them

via the SAVE (F8) function.

.P

MONITOR TYPE (Default = Mono)                                          9701

Use  the UP/DOWN arrow keys to select either a MONOCHROME or COLOR  display

monitor.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

TEXT (Default = Yellow)                                                9702

Use the UP/DOWN arrow keys to select the desired color for screen TEXT.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

STATUS LINE (Default = White)                                          9703

Use  the UP/DOWN arrow keys to select the desired color for the STATUS LINE

located in the lower portion of the top right-side window.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

MESSAGE LINE (Default = White)                                         9704

Use the UP/DOWN arrow keys to select the desired color for the MESSAGE LINE

located in the upper portion of the top right-side window.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

HIGHLIGHT (Default = White)                                            9705

Use the UP/DOWN arrow keys to select the desired color for the  HIGHLIGHTED

screen text.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

BACKGROUND (Default = Blue)                                            9706

Use  the  UP/DOWN  arrow keys to select the desired color  for  the  screen

BACKGROUND.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

SCREEN OUTLINE (Default = Lt Red)                                      9707

Use  the  UP/DOWN  arrow  keys to select the desired color  of  the  screen

OUTLINE.

Screen configuration changes must be SAVED (F8) before you EXIT (F10)  this

screen.

.P

CHANGE/VIEW RADIO-WIDE CONFIGURATION                                   4200

RADIO-WIDE  Configuration  is  used to  CHANGE/VIEW  radio  parameters  and

options that affect overall radio operation, i.e. those not just related to

a specific channel such as the time-out-timer.

Warning:  CHANGE/VIEW does NOT actually modify the radio codeplug data, but

instead  it  modifies  a  copy of the  data  retrieved  from  the

codeplug (or archive file) via the GET/SAVE functions.

After  all  CHANGE/VIEW modifications  are  completed,  you  MUST

return to the GET/SAVE menu and PROGRAM the changes back into the

radio  or  SAVE  them  to a  new  archive  file.   Otherwise  the

modifications will be lost when you turn off your computer.

.P

VOX SENSITIVITY                                                        4201

This is normally a factory adjustment and does not need to be changed.  If,

however, you wish to decrease the VOX Sensitivity (make it open for louder

sounds), then increase the value here.

.P

ACCESSORIES                                                            4202

You may choose to have your accessory connector behave in one of two ways,

"auto  sense"  or "headset only". "Auto Sense" will sample  the  microphone

jack  each time the radio is turned on. If a PTT input is sensed, then  the

radio will assume that the accessory is a headset. All subsequent  external

PTT  inputs  will be ignored. If no PTT is sensed when the radio  is  first

turned  on, then the accessory is assumed to have it's own PTT switch,  for

example a speaker microphone.  If you choose "headset only", then the radio

will assume that it can ignore all external PTT inputs. Thus, only a  head-

set  or earpiece can be used with the radio. Speaker microphones  will  not

function if this choice is selected.

.P

TIME-OUT-TIMER (TOT)                                                   4203

If Enabled, the TOT will limit the maximum time allowed for any  continuous

transmission  from  1  to 255 seconds on all  transmit  channels.   Receive

operation is not affected by the TOT.

Use  the UP/DOWN arrow keys to select the desired time.  You will  see  the

display  change  in  one second increments.  The data  field  may  also  be

changed  by typing the desired time or "OFF" from the keyboard.  The  range

is from OFF to 255 seconds.

.P

CONTROL BUTTONS                                                        4204

There are two control buttons  on the side of the radio, just above the PTT

switch.  Control button 1  is the top button, while control button 2 is the

bottom button.   Each control button  may be programmed  to have a specific

function.   The choices are Volume  Set, Monitor, Nuisance Delete and None.

The default  values are  Volume Set on  control  button 1  and Monitor  on

control button 2.

Radios  with   keypads have  four additional   control  buttons  which  are

programmable.  Please refer to the  Radio Service Software  manual  for the

location  of these  buttons.    The  choices  are Monitor, Nuisance Delete,

Exit Phone, and None.  Currently, the choices of Phone and Memory are

forced on control buttons 3 and 4, respectively and are not allowed on

the other control buttons.

.P

OPTION 2 BUTTON                                                        4205

.P

ALERT TONES                                                            4206

This field determines  if  alert tones are enabled or disabled.   The alert

tones affected by this field  are Power Up,  PTT Sidetone,  Time Out Timer,

Busy Channel, Talk Prohibit,  Call Alert, Voice Sel Cal, Low Battery Alert,

Scan Talkback,  Priority Channel, Scan Activate, Codeplug Failure and Good/

Bad Key Chirp.

.P

LIGHTS                                                                 4207

All on  - Enables all indicator LEDs except for Channel Busy Light

All off - No LEDs will be seen

.P

CHANNEL BUSY LIGHT                                                     4208

This field determines if the Channel Busy Light is enabled or disabled.

Some reasons why you may which to disable the Channel Busy Light are:

1. User could confuse the indicator with the low battery transmit light.

2. Increases current drain on heavily used community repeater channels.

3. Extraneous information if channel is carrier squelch.

4. Extraneous information if Busy Channel Lockout is used.

.P

LOW BATTERY                                                            4209

This field  determines the type of  Low Battery alert.   The radio can give

several different types of feedback to the operator when the battery is low

and  needs  to be  recharged.   There  are choices of  no  feedback  (OFF),

flashing transmit light when keyed (LIGHT), and an audible tone in receive/

flashing transmit light when keyed (LGHT/TONE).

The Tones and Lights fields influence what choices are available for this

field.

.P

LOW BATTERY (cont)

The rules are as follows:

TONES        LIGHTS         LOW BATTERY

all on       all on         lght/tone, off

all off      all on         light, off

all on       all off        tone, off

all off      all off        off

.P

FORCED MONITOR                                                         4210

This  field determines  if  Forced Monitor is  enabled  or disabled.   When

enabled,  this  feature forces  the radio  to be  in monitor  (PL disabled)

before  the radio can be  keyed.  The operator will hear a low pitched tone

if PTT is pressed when not in monitor and the monitor light will illuminate.

Pressing PTT a second time will cause the radio to key.

Note:  We  DO NOT recommend  using this  feature  in combination with VOX,

Busy Channel Lockout or TX Inhibit.

.P

TIME-OUT-TIMER (TOT) REKEY TIME                                        4211

If enabled, this will be the amount of time that needs to elapse from  the

expiration of the time-out-timer (tot) until a retransmition can occur.  If

no elapsed time is needed or necessary, then disable this function. The

allowable input values are 6.0 seconds or "OFF" to disable.

.P

LOW BATTERY THRESHOLD                                                  4212

This field displays the low battery threshold level of the radio.  The low

battery threshold level is the voltage level at which the radio will give

alert tones and lights.  Due to different battery voltage characteristics,

this value may require adjustment if the radio is equipped with a HNN9701 FM

approved battery.  If you are not getting adequate battery life with a

HNN9701 battery, contact product services for an adjustment procedure.

.P

CHANGE/VIEW MODE CONFIGURATION                                         4500

MODE Configuration is used to CHANGE/VIEW channel specific information such

as frequencies, squelch type, squelch codes, and scan lists.

Warning:  CHANGE/VIEW does NOT actually modify the radio codeplug data, but

instead  it  modifies  a  copy of the  data  retrieved  from  the

codeplug (or archive file) via the GET/SAVE functions.

After  all  CHANGE/VIEW modifications  are  completed,  you  MUST

return to the GET/SAVE menu and PROGRAM the changes back into the

radio  or  SAVE  them  to a  new  archive  file.   Otherwise  the

modifications will be lost when you turn off your computer.

.P

NAME                                                                   4501

This field allows the user to indicate  mode placement.   The mode name can

be any value  between  01 and the maximum number of  channels  available in

the radio.

The radio will come defaulted from the factory with  a few test channels in

the first  positions of  the rotary  switch.  You  may "fill" in unused

rotary positions with this field.  Naming more than one mode with the same

name will cause the two  positions to be "strapped together."   Refer to

the Radio Service Software manual's section on Mode Placement Choices for

further information and examples.

.P

TYPE                                                                   4502

This field indicates the mode type.  The available types are conventional

and scan.

.P

RECEIVE FREQUENCY                                                      4503

Enter  the RECEIVE frequency directly in MHz.

Blank RECEIVE frequencies are NOT allowed.

Refer  to the MOTOROLA Catalog Sheets / Price Pages or your Service  Manual

for valid RECEIVE frequencies for this model.

.P

TRANSMIT FREQUENCY                                                     4504

Enter  the TRANSMIT frequency directly in MHz.

For a "Receive-Only" channel,  enter a B.   The TRANSMIT frequency will  be

displayed as BLANK.

Refer  to the MOTOROLA Catalog Sheets / Price Pages or your Service  Manual

for valid TRANSMIT frequencies for this model.

.P

MODE                                                                   4505

This function allows the user to go directly to a specific mode.

The default is the last mode displayed.  Choices are 1 through the

total number of modes in the current configuration.

Important note:  The RSS will blank out the Mode field and allow the

user to input a mode number.  If the number is an invalid choice or is

left blank, the RSS will will revert back to the last mode displayed.

Otherwise, the specified mode will be displayed.

.P

SQUELCH TYPE                                                           4507

Use  the UP/DOWN arrow keys to select the SQUELCH type:

CSQ - Carrier Squelch

TPL - Private Line

DPL - Digital Private Line

INV.DPL - Inverse DPL

Data  fields  for transmit and receive codes will be displayed for  PL  and

DPL.

.P

PL SQUELCH CODE                                                        4508

Enter  the  PL  frequency  directly in  Hz  (Ex:   192.8),   or  enter  the

alphanumeric code (Ex:  7A).

Enter a B for Carrier Squelch operation.

Valid codes may also be selected via the UP/DOWN arrow keys.

Standard PL frequencies are listed on the next page:

.P

PL SQUELCH CODE (cont)

Standard Private Line Frequencies / MOTOROLA Code:

Freq  Code    Freq  Code     Freq  Code     Freq  Code     Freq  Code

67.0   XZ     91.5   ZZ      123.0  3Z      167.9  6Z      225.7  M4

69.3   WZ     94.8   ZA      127.3  3A      173.8  6A      229.1  9Z

71.9   XA     97.4   ZB      131.8  3B      179.9  6B      233.6  M5

74.4   WA     100.0  1Z      136.5  4Z      186.2  7Z      241.8  M6

77.0   XB     103.5  1A      141.3  4A      192.8  7A      250.3  M7

79.7   WB     107.2  1B      146.2  4B      203.5  M1

82.5   YZ     110.9  2Z      151.4  5Z      206.5  8Z

85.4   YA     114.8  2A      156.7  5A      210.7  M2

88.5   YB     118.8  2B      162.2  5B      218.1  M3

.P

DPL SQUELCH CODE                                                       4509

Enter  the  DPL  code  directly (Ex:  031);  note that  leading  zeros  are

required.

Enter a B for Carrier Squelch operation.

Valid codes may also be selected via the UP/DOWN arrow keys.

Valid DPL codes are listed on the next page:

.P

DPL SQUELCH CODE (cont)

Valid DPL Codes:

Code   Code    Code    Code    Code    Code    Code    Code    Code

023    071     134     223     306     411     503     631     734

025    072     143     226     311     412     506     632     743

026    073     152     243     315     413     516     654     754

031    074     155     244     331     423     532     662

032    114     156     245     343     431     546     664

043    115     162     251     346     432     565     703

047    116     165     261     351     445     606     712

051    125     172     263     364     464     612     723

054    131     174     265     365     465     624     731

065    132     205     271     371     466     627     732

.P

RX Signalling System                                                   4510

Enter the number of the Signalling System that is desired for the  receiver

of  this  mode. Signalling Systems are set-up under the  Signalling  System

Configuration Screen.

If there is no Signalling System for this mode enter "00".

.P

TX Signalling System                                                   4511

Enter  the  number  of  the  Signalling  System that  is  desired  for  the

transmitter of this mode. Signalling Systems are set-up under the

Signalling System Configuration Screen.

If there is no Signalling System for this mode enter "00".

.P

BUSY CHANNEL LOCKOUT                                                   4512

Busy  Channel Lockout may be enabled by choosing "Y".  When enabled,  the

radio will be 'polite' to other users on the channel.  In other words,  the

radio will not let you hear conversations of other users and will not allow

you  to transmit on top of another user.  The only calls you will hear  are

those  with  your PL code,  even if you press  the  monitor

button.  If  you  press PTT on the radio while there is a  carrier  on  the

channel, the radio follows this protocol:

a. If the carrier has your PL, then you are allowed to transmit

just like normal.

.P

b. If the carrier has no PL or the wrong PL, then you will hear

a  busy  tone while you hold PTT.  Release PTT and  try  again

later.

If you choose "N", then the radio will transmit and receive like a normal

two-way radio.

.P

VOX                                                                    4513

This field determines if the Voice Operated Transmission is enabled or

disabled.  If enabled, VOX requires an accessory headset that has a Mic

audio live at all times.

The radio will sense the presence of the headset each time the radio is

turned on.  You may, therefore, defeat VOX by removing the headset and

cycling the power switch.

When channel scan is enabled, the radio's software must decide as to which

way VOX will behave.  This decision will supercede all other mode

specifications regarding VOX.

.P

BATTERY SAVER                                                          4514

This field sets the rate of the Battery Saver feature.  The possible choices

for this field are Short, Medium, Long or Off.

The following is a summary of characteristics for the three rates:

RATE      RX On Time    Sleep Time     Improved Capacity

Short      50 ms           50 ms             8%

Medium     50 ms          150 ms            15%

Long       50 ms          250 ms            20%

.P

POWER                                                                  4515

This field sets the  power level.   The choices are high power (4 or 5 W),

medium power (2 W) or  low power (1 W).  Your  radio is  calibrated at the

factory for each of  these power  levels.   Your particular  model  may be

calibrated to different power levels due to local regulatory requirements.

Consult your Service Manual for more information.

.P

SCAN LIST NUMBER                                                       4521

This  field indicates  the number of the  scan list that is  assigned to be

used with this mode.

.P

PRIORITY SAMPLING RATE                                                 4522

When the channel scan is stopped on a non-priority channel, it periodically

"looks back" at the priority channel for activity.  The rate at which  this

"look  back"  occurs  is  the priority sample  rate.   The  are  two  rates

available,  slow  and fast.  Slow is 1.5 seconds between "look  backs"  and

Fast is 0.5 seconds between "look backs".  Slow is the default.  To  change

the sample rate use the up and down arrow keys to select the rate.

Note : Earlier radio software versions do not support this feature!

.P

SCAN HANG TIME                                                         4523

This field  sets the  length of  time that  the radio  will  remain  on the

channel  after receiving the  end of a transmission.   The hang time should

be as short as necessary  to maintain  message continuity.   The default is

2.5 seconds  and  the values can  range  from 200 ms to 4.2 s.  If the scan

list  is all non-priority channels,  values  between 33 and  200 ms  can be

used. Scan lists with  priority channels  CANNOT  have zero hang time.

.P

HOME REVERT MODE                                                       4524

This field sets the Home Revert Mode for this channel.

If  the TalkBack  feature is  enabled, then  the  radio  will act  in the

following  manner.  If you hear activity on  a mode, then  the radio will

stop on this active mode until the  hang timer expires.  During this hang

time any transmission will occur on this active mode.  In all other cases,

the radio will revert to the Home Mode.

If  TalkBack is  disabled,   then the  radio  will always  revert  to the

preprogrammed Homd Mode for transmissions,   regardless  of hang time.

.P

HOME REVERT MODE (cont)

The  scan will  default with TalkBack enabled when a scan list is created.

The Home Revert Mode  can be specified as any conventional channel in the

radio.  A good practice  is to  leave this set at the default priority 1.

.P

SCAN TALK BACK                                                         4525

The conventional channel scan has the capability to revert to the home mode

(home  revert) or stay on the busy channel (talk back) when the  microphone

is removed from the hang-up clip.  These capabilities are now  programmable

from  Radio  Service  Software.  The default is  home  revert.   To  change

capabilities  use the up and down arrow keys to select "N" for home  revert

scan and "Y" for talk back scan.

Note: Earlier radio software versions do not support this feature!.

.P

ACTIVE CHANNEL TONE                                                    4526

This field will determine if the Active Channel Tone is enabled or disabled.

If enabled, Active Channel  Tone will allow the  user to rotate the channel

selector to  find  the last active  channel.   When the channel is found, a

single tone  will  be heard.   The tone  will remain  associated with  that

channel until the radio is powered off/on, or the PTT is pressed.

.P

PRIORITY CHANNEL TONE                                                  4527

This field  determines if the Priority Channel Tone is enabled or disabled.

If enabled, a tone will be produced whenever the  radio scans to  and stops

on a priority channel.  This alerts the operator that a priority message is

being  listened to.  It is suggested that this tone be turned off if there

is heavy use of RapidCall calling features on the priority channel.

.P

SCAN ACTIVATE TONE                                                     4528

This field determines if the Scan Activate Tone is enabled or disabled.  If

enabled, a tone will be  produced when the rotary  selector  is turned to a

scan list  position.  This is helpful to the user when the radio is worn on

a belt.

.P

PHONE SIGNALLING SYSTEM                                                4529

Enter the number of the Signalling System that is desired for use when

performing phone dialing operations on this mode.

.P

TX INHIBIT                                                             4530

This feature is only available when the channel has a CSQ receive code.

When enabled, this feature will not allow the radio to transmit while a

carrier is present on the channel.  A busy tone will be heard in such a case.

.P

MODE SLAVED SCAN LIST                                                  4561

This  screen displays the available modes and their scan TYPE for the  mode

display in the upper left side of the display.  The scan types are  changed

by  moving  the cursor with the Enter or Tab keys to the desired  Mode  and

changing its TYPE by using the up and down arrow keys.  Note that  Priority

One  is  always  the Mode that the current scan list is  slaved  to.   This

priority cannot be changed.

Function Key Descriptions:

F2 -  GO TO MODE allows you to go to the Mode Slaved Scan List you want  to

modify  without  returning  to  the  MODE  CONFIGURATION  screen  and

stepping through the modes in-between.

.P

MODE SLAVED SCAN LIST (cont)

F3 & F4  - NEXT MODE will advance the screen to the next Mode  Slaved  Scan

List.  If you are at the last Mode in the radio then pressing F4 will

take  you to Mode 1.  PREVIOUS MODE will return you to  the  previous

Mode.   If you are at Mode 1, pressing F3 will take you to  the  last

Mode in the Radio.

F5 -  PRINT SCREEN will send the currently displayed Mode Slaved Scan  List

to the printer.

F7 -  ERASE LIST will remove all of the Modes from the current scan list.

.P

MODE UTILITY                                                           4580

The  MODE  UTILITY  screen is used for  adding,  deleting,  or  moving  the

location  (order)  of  Conventional  modes.    Modes  may  also  be  copied

(duplicated), including scan lists (Mode-Slaved only).

The  F8  (EXECUTE)  function key is used to execute the  changes  once  all

information has been entered.

The  GET/SAVE functions must be used to program the changes into the radio,

as is required for all CHANGE/VIEW modifications.

.P

UTILITY (Default = Add Mode)                                           4581

Use  the  UP/DOWN arrow keys to select the desired UTILITY  function:   Add

Mode, Copy Mode, Delete Mode, or Move Mode.

The  F8  (EXECUTE)  function key is used to execute the  changes  once  all

information has been entered.

The  GET/SAVE functions must be used to program the changes into the radio,

as is required for all CHANGE/VIEW modifications.

.P

MODE TYPE                                                              4582

Mode Type is Conventional and cannot be changed.

.P

INSERT AFTER MODE                                                      4583

Enter the mode number that the mode is to be INSERTED AFTER.

After entering this number and pressing Enter,  the mode name for that mode

will  be  displayed.   Use  the name to verify that the proper  number  was

entered.

.P

NEW MODE NAME                                                          4584

Enter the 8-character alpha-numeric MODE NAME.   The Mode Name will default

to "Mode nn", where nn = mode number.  If Mode Names are disabled, you will

not be able to change this field.

Each  mode  may be assigned a Name to facilitate ease  of  operation.   The

[Mode] rocker button is used to scroll forward or backward through the list

of modes programmed in the radio.

Since  the mode number is required for direct keypad mode selection via the

[Home] button for keypad models,  using the mode number as part of the mode

name is desirable, but not required.

.P

MODE TO COPY FROM                                                      4585

Enter the Mode Number TO COPY FROM.  Any valid conventional mode number may

be entered.

After entering this number and pressing Enter,  the mode name for that mode

will  be  displayed.   Use  the name to verify that the proper  number  was

entered.

Frequencies  (Rx,  Tx,  T/A),  squelch type,  squelch codes,  and all  mode

options  will  be copied.   You may choose to copy the  channel  scan  list

(mode-slaved  only)  or  to enter a new one.

This  field is required for the Copy Mode function;  it is optional for the

Add Mode function.

.P

COPY SCAN LIST (Default = N)                                           4586

Use the UP/DOWN arrow keys to select Y to copy the channel scan list (mode-

slaved only) or N for a blank list.

If  the  copied  list  is full (16 modes),  the new mode  will  assume  the

priority of the copied mode and the copied mode will not be part of the new

list.

Otherwise,  the new mode will assume the priority of the copied  mode,  and

the copied mode will be a non-priority mode on the new list.

.P

MODE TO COPY TO (Default = Blank)                                      4587

Enter  the Mode Number TO COPY TO.

After entering this number and pressing Enter,  the mode name for that mode

will  be  displayed.   Use  the name to verify that the proper  number  was

entered.

Frequencies  (Rx,  Tx,  T/A),  squelch type,  squelch codes,  and all  mode

options  will  be copied.

.P

MODE TO DELETE                                                         4588

Enter the Mode Number TO DELETE.

After entering this number and pressing Enter,  the mode name for that mode

will  be  displayed.   Use  the name to verify that the proper  number  was

entered.

CAUTION:  After  the F8 (EXECUTE) function key has been pressed,  the  mode

will be deleted and may not be recovered.  However, if you make a

mistake  AND the data has not been programmed or saved  (GET/SAVE

functions), you may re-read the codeplug or original archive file

and start over.

.P

MODE TO MOVE                                                           4589

Enter  the  Mode Number TO MOVE.   Any valid conventional mode  number  may

moved.

After entering this number and pressing Enter,  the mode name for that mode

will  be  displayed.   Use  the name to verify that the proper  number  was

entered.

The mode and all the releated parameters will be moved to the new position.

.P

PHONE MEMORY DIALER                                                    4600

This screen allows the user to create a list of phone numbers.  Up to nine

16-DTMF-digit phone numbers can be entered on this screen.

.P

SYSTEM                                                                 4700

This  screen  allows  you to  configure one  or  more  signalling  systems.

These  signalling systems will be used by the Call List and Mode screens to

determine when and where decode or encode.

.P

TYPE                                                                   4702

When this screen is displayed the cursor is placed on the TYPE data  field.

The  available  system  types  are None,  MDC-1200,  QCII (QUIK CALL), DTMF

and DTMF/PHONE  (only  on phone capable radios).    On  Limited Signalling

models the available  system  types are  None, MDC1200-PTTID,  MDC1200-DOS,

QCII and DTMF/PHONE (only on phone  capable radios).  The system types  are

selected  by  using the Up/Down arrow keys.    None implies that all of the

signalling  systems in the radio are  deactivated.

.P

ID                                                                     4703

MDC-1200, MDC1200-PTTID :

MDC-1200 requires that each radio have a unique four digit ID number. The

default is the last four digits of the radio serial number.  The range is

from 0001 to DDDD.

QCII :

QCII ID corresponds to tones A and B, each tone must be unique.

The defaults are 358.6 Hz (tone A) and  903.2 Hz (tone B).   The range is

from 288.0 to 3100.0 Hz.  Don't be alarmed if the frequency that you type

in changes by a few Hertz. This change can occur because the program must

calculate  the nearest  frequency that  the radio  can  understand.   You

will find that the error  will always be  within the acceptable  industry

bandwidth.

DTMF :

DTMF  requires each radio to have a unique 1 to 8 digit ID  number.   The

default is the last four digits of the radio serial number.   Digits 0 to

9, A, B, D, *, and # may be used.  Any unused digits may be filled in

with a '-' character.  It is recommended not to use A, B, and D.

NOTE: Avoid having the same digit listed sequentially in an ID.  This

invites potential false decode problems.  For example, an ID of

1134 will have a greater probability of false decode than 1234.

.P

GROUP ID                                                               4704

The  MDC1200 ID is a three hex digit number common to a group  of  radios

and ranges from 000 to EEE.

.P

CALL FORMAT                                                            4705

The sequence in which the radio needs to receive the QCII tones for an

individual or group call.

Default:  A-B

Choices: A-B            A-B/A-C/Long C

A-B / A-C      A-B/Long B & C

A-B / C-B      A-B/A-C/Lg B&C

A-B / Long B   A-B/A-D/C-D

A-B / Long C

.P

IMPORTANT NOTE:

The CALL FORMAT choice was not required or available for some older version

radios. Older version GP300 radios do not support Monitor-style tone C

operation.

.P

CALL TYPE                                                              4706

This field determines what response a radio has when selectively called.

Choices: Call Alert(default), Call Alert/Voice, Voice Sel Cal

Call Alert leaves a visual/audible indication ONLY.  The visual/audible

indications repeat until any button is pressed on the radio.

Call Alert/Voice leaves a visual/audible indication and unmutes the radio to

leave an audible message. Both the light/tone indications repeat until any

button is pressed on the radio.

.P

CALL TYPE (cont)

Voice Sel Cal leaves a one-time, two-beep tone and the LED flashes yellow.

The radio will then unmute and a voice message should follow.  The yellow

LED will continue to flash for the entire length of the message and return

to normal operation when the message ends.  For QCII (Quick Call), this is

a selectable option.  For MDC-1200 signalling, Voice Sel Cal is inherent,

so the radio will receive Sel Cals with the radio set to either Call Alert

or Call Alert/Voice.

.P

PL/DPL REQUIRED                                                        4707

The Up/Down arrow keys will allow the choices of either "Y" or "N".   "Y"

implies the correct PL/DPL is required to recieve data.  "N" implies PL/DPL

is  not  required  to receive data.  In most systems you  may  choose  "N",

however,  when  you  share  a channel with other users who  have  the  same

signalling type then you should choose "Y". Choosing "Y" on shared channels

will ensure that you don't accidentally receive a call which is directed to

another user.

Note :  Some existing DTMF and Quik Call systems require PL stripping, and

therefore it is recommended that an "N" be chosen in these cases.

.P

ALERT TONE RESET                                                       4708

Alert Tone Reset only applies if the Call Type is CALL ALERT or CALL ALERT

W/VOICE.  This determines how the audible alert tone will be reset.  The

choice of AUTOMATIC means that the alert tone will be automatically reset

for you.  The choice of MANUAL means that there will be repeating alert

tones every 10 seconds that must be manually reset by pressing any button.

.P

CONVERSATION RESET                                                     4709

Conversation Reset only applies to a Call Type of CALL ALERT W/VOICE.  When

a call alert is received you may wish the radio remain in CSQ operation for

a follow-up voice message or so that you may hold a brief conversation.  At

the end of the conversation, you may also want the radio to return to its

previous squelch condition.  If you choose MANUAL RESET, then the radio

will reset the squelch condition immediately after the operator presses the

monitor button.  If the choice of AUTO RESET is selected, then the radio

will wait for seven seconds after a carrier loss and/or radio transmission

before resetting the squelch condition.

.P

ACK PRETIME                                                            4711

Systems   that  use Call Alert require that the radio  acknowledge  (ACK)

that the  Call Alert has been received.  ACK Pretime is the delay from when

the  radio  receives  a  Call Alert to when it starts  the  ACK   sequence.

The default ACK pretime is 0.5 seconds.  If a different pretime is required

for your  system move the cursor to the ACK Pretime data field and   change

the time  using  the up and down arrow keys.  The range can be from 0.5  to

2.1 seconds in 0.1 second steps.

.P

PTT DISABLES SIG SQ                                                    4713

If this feature is enabled "Y", then the signalling squelch feature will

be disabled for a fixed period of time after the PTT button is pressed.

This allows other users to reply to your call without having to send you a

Call Alert or Sel Cal.  The default is "N" for disabled.  This feature is

only available and meaningful when signalling squelch is enabled.

.P

SIGNALLING SQUELCH                                                     4714

This  feature can either be enabled "Y" or disabled "N" using the Up/Down

arrow  keys.  If this feature is enabled "Y",  then one of the control but-

tons  can  be used to put the radio into  signalling  squelch.   If  this

feature  is  disabled "N",  then signalling squelch is not  activated.  The

default is "N".

Signalling  Squelch is similar to Private Line.  When enabled,  the radio

will  only unmute for Call Alerts and Voice Sel Cals.  Like  Private  Line,

Signalling  Squelch  can  be  disabled by pressing the  monitor  button

You   will   find  that Signalling Squelch is  automatically  ENABLED  when

channel scan is on.

.P

DOS HOLDOFF                                                            4715

Data   Operated   Squelch  (DOS)  is  used with  data signalling  systems

(such  as  MDC-1200)   to    mute    the  data  transmissions   to   reduce

operator  annoyance.   DOS  attempts  to  mute  the  speaker  when data  is

present  on  the channel.    DOS Holdoff keeps  the speaker muted   for   a

preset period  of time from when carrier is  detected   by    the   squelch

circuit.    This  allows the DOS time to detect any data  activity, further

reducing the annoyance of data being heard from the speaker.

The  default  holdoff  is 0.0 seconds.    If a different holdoff time  is

required  for   your system  move the cursor to the DOS Holdoff data  field

and  change the   time using the Up/Down arrow keys.  The range can be from

0.0   to  4.2  seconds  in  0.1 second  steps.

.P

PTT ID                                                                 4716

If you want your unit id to be sent on each Push-to-talk (PTT),  then set

this field to "PRE", "POST", or "NO".  "PRE" sends the id at the press of

PTT.  "POST" sends the id upon release of PTT. If  you choose "NO",  then

the id will not be sent during PTT.

.P

PTT SIDETONE                                                           4717

PTT   sidetones   are  used when PTT ID is  activated.    Sidetones   are

heard  through the speaker when the operator first presses the  PTT.   They

are  used  to  tell  the  operator   that he cannot  speak  until  the   ID

has   been transmitted.  There are two sidetones available in the  Radius.

The  PTT  ID Sidetone  is a continuous tone heard from PTT until the ID has

been  sent.  The second tone, PTT Short Sidetone, is a short beep after the

ID  has   been sent.    Both  tell  the operator when he is  permitted   to

speak  into  the microphone.   One,  both  or no Sidetones maybe activated.

They   can  be activated  by moving the cursor to the desired sidetone  and

using the Up/Down arrow keys to choose either "Y" or "N" to either activate

or deactivate the sidetones.

.P

PTT SHORT SIDETONE                                                     4718

PTT   sidetones   are  used when PTT ID is  activated.    Sidetones   are

heard  through the speaker when the operator first presses the  PTT.   They

are  used to  tell  the  operator  that he cannot yet speak until  the   ID

has   been transmitted.  There are two sidetones available in the  Radius.

The PTT  ID Sidetone  is a continuous tone heard from PTT until the ID  has

been  sent.  The second tone, PTT Short Sidetone, is a short beep after the

ID  has   been sent.    Both  tell  the operator when he is  permitted   to

speak  into  the microphone.   One,  both  or no Sidetones maybe activated.

They   can  be activated  by moving the cursor to the desired sidetone  and

using  the  Up/Down arrow keys to choose either "Y" or "N" to  activate  or

deactivate the sidetones.

.P

PRETIME                                                                4719

The    pretime    is  the  delay from the start  of  transmit  (when  the

operator pushes PTT) to when  the data signalling begins.    This delay  is

required   in  order  to  allow   the  radio    to   completely   open  the

communications  path  through   repeaters  or  other   system   components.

The  default   pretime   is 0.5 seconds.    If  a  different   pretime   is

required  for your  system move the cursor  to  the Pretime data field  and

change the  time using the Up/Down arrow keys.   The range can be from  0.1

to 4.2 seconds in 0.1 second steps.

.P

FIXED RETRY WAIT                                                       4720

When  emergency  is  activated  by  the  operator  the  radio  expects an

acknowledge from the base station.  If no acknowledge is received then  the

radio   will repeat the emergency sequence after a delay.  The radio always

waits a random time greater than twice the Pretime.  The additional time is

the  Fixed Retry Wait time.  Fixed Retry Wait is the time the  radio  waits

between emergency tries.  The default  is 0.0  seconds (STAR default is 8.1

seconds).   If  a  different Wait Time is required move the cursor to   the

Fixed  Retry  Wait data field and change the time using the  Up/Down  arrow

keys.   The  range  can be from 0.0 seconds to 16.9 seconds in  0.4  second

steps.

.P

TX TONE DURATION                                                       4723

This is the length of each digit that is transmitted by the  radio.

Use the Up/Down arrow keys to change the default value.

For DTMF:

The range is 49.5 to 4191.0 msec.  in 16.5 msec. intervals.

The default is 99.0 msec.

For DTMF/PHONE:

The range is 66.0 to 4191.0 msec. in 16.5 msec. intervals.

The default is 297.0 msec.

.P

TX TONE INTERVAL                                                       4724

This is the time between the transmission of different DTMF digits.   The

default  is 66.0 msec.   Use the Up/Down arrow keys to change the  default

value.  The range is 49.5 to 4191 msec. in 16.5 msec. intervals.

.P

ACKNOWLEDGE                                                            4725

If Acknowledge is enabled, then the radio will transmit a transpond signal

when it receives the correct ID.

.P

ACCESS CODE                                                            4726

A string of up to 8 DTMF digits that will be used to access a phone patch

when the MEM * sequence is pressed on a mode where DTMF/Phone is assigned.

Default: *-------

Choices: 1 to 8 DTMF digits(0-9,*,#,A,B,D,-)

- means unused.

A,B,D are not recommended.

.P

DEACCESS CODE                                                          4727

Aa string of up to 8 DTMF digits that will be used to disconnect a phone

patch when the MEM # sequence is pressed while in phone mode.

Default: #-------

Choices: 1 to 8 DTMF digits(0-9,*,#,A,B,D,-)

- means unused.

A,B,D are not recommended.

.P

SIDETONE                                                               4728

The Sidetone is a continuous tone heard from  PTT until the ID has been sent.

Sidetones are heard through the speaker when the Access and Deaccess Codes

are being transmitted.  They are used to tell the operator that he cannot

yet speak until the ID has been transmitted.

.P

SHORT SIDETONE                                                         4729

The Short Sidetone is a short beep after the ID has been sent.

Sidetones are heard through the speaker when the Access and Deaccess Codes

are being transmitted.  They are used to tell the operator that he cannot

yet speak until the ID has been transmitted.

.P

TX HOLD TIME                                                           4730

The Tx Hold Time determines how long the radio will remain keyed between

keypad presses.  This allows the transmitter to stay up when using the DTMF

keypad for "live" dialing.  The minimum value for this field is determined

by the values of Tx Tone Duration and Tx Tone Interval.  The Tx Hold Time

CANNOT be less than the sum of the two Tx Tone values.  The maximum value

for this field is 49.5 to 4191 msec.

.P

OVERRIDE BCL/PL                                                        4731

Allows Busy Channel Lockout and PL rules to be modified when the radio is

in phone mode.

Default: N

If the answer is (Y), the system overrides the BCL rules and strips PL/DPL

everytime the user enters phone mode.  This behavior is desirable for PL/DPL

repeater-based phone patches that strip PL/DPL during phone calls. If the

answer is (N), the system will either follow BCL rules if BCL is enabled on

the mode, or if BCL is not enabled, then PL/DPL rules will be followed.

.P

033333333§33Ä333333

33”3333§3333ÿ3333 33µ33Standard      Radius        MaxTrac       Trunked       Voice_ReporterFlexar_fill   Radius_Int'l  RadiusReporterFlex Indirect Rad Low Sig   Max Low Sig   Rad I Low Sig Rad High Sig  Max High Sig  Rad I High SigMax Options   Rad Options   Max Trk OptionRad I Option  Radius TrunkedRadius GP300  Radius GP300kmRadius GP300gs33333333333333333333333333333333333333333333333333333333Radius GS300kmRadius GS300gsRad EuroMobileRadius P110   People Finder 333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333Radius GP350xx33333333333333333333333333333333333333333333333333333333Links/SysTel  3333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333333Radius GP350  Radius M100   Radius M206   Radius M214   VR100         VR100 TRUNKED MaxTrac 50    MaxTrac 100   MaxTrac 300   MaxTrac 820   MaxTrac 840   Privacy Plus  VR100         VR100 Trunked Dual Mode     Smartnet 840  Radius M100 I Radius M214 I Radius M206 I P. Plus 250   P. Plus 750   Smartnet 2000 Smartnet C3   Smartnet C5   Radius M800   Radius M805   Radius GP300  Radius GP300kmRadius GP300gsRadius GS300kmRadius GS300gsRadius P110   People Finder 3434343434343434343434343434Radius GP350xxLinks/SysTel  Radius GP350  äTäTäT34”\”\”\34PTPTP^34´Ì´Ì´Ì34¾¤ê¾¤ê¾¤ê34\\

cd \MRSS\GP350

GP350.EXE

C:

cd \MRSS\GP350

GP350.EXE

3434¤